Innopolis University

Master's Programme in Computer Science

AI-Assisted Simulation and Optimization of Power Market Projects

Master's Thesis

Author: Dadakhon Turgunboev

Supervisor: Leonard Johard

Innopolis, 2026

Table of Contents

Abstract	8

Introduction	9

Relevance of the Research	9

Problem Statement	10

Object and Subject of Research	11

Purpose of the Research	11

Research Tasks	11

Scientific Novelty	12

Practical Significance	13

Thesis Structure	13

Literature Review	14

Power System Analysis and the PyPSA Framework	14

Julia for Scientific Computing and Optimization	15

Existing Julia Tools for Power System Analysis	16

Short-Term Load Forecasting	17

Conformal Prediction	18

Stochastic Optimization in Power Systems	19

Research Gap	20

Design and Methodology	21

DC Power Flow: Mathematical Foundation	21

Power System Model	21

Susceptance Matrix Construction	21

Slack Bus and Reduced System	22

Active Power Flows	22

AC Power Flow: Mathematical Foundation	23

Full AC Power Flow Equations	23

Newton-Raphson Solution Method	23

Per-Unit System	24

PowerModels.jl Integration	24

Linearized AC Power Flow: Mathematical Foundation	25

Motivation and Position in the Hierarchy	25

First-Order Taylor Linearisation	25

Comparison with DC Power Flow	26

Linear Optimal Power Flow: Mathematical Foundation	26

Problem Formulation	26

Connection to DC Power Flow	27

LP Solver Interface Design via JuMP	27

Technology Stack and Tool Selection	27

Julia Ecosystem	27

Python Ecosystem	28

Comparative Analysis of Ecosystems	28

Benchmark Methodology	29

Random Network Generator	29

JIT Warmup Strategy	30

Python Timing Methodology	30

Hardware and Software Environment	30

Unit Commitment: Mathematical Foundation	30

AI Component: LSTM Load Forecasting	31

Motivation	31

LSTM Architecture	32

Training Protocol	32

Conformal Prediction Intervals	32

Stochastic LOPF via Sample Average Approximation	33

Uncertainty Model	33

Sample Average Approximation	33

Implementation and Experiments	35

Test Network Definition	35

Core Implementation	36

Validation	37

AC Power Flow Implementation	39

PowerModels.jl Data Format	39

Implementation	40

Branch Flow Extraction	41

Validation	42

Linearized AC Power Flow Implementation	43

Implementation	43

Validation	44

LOPF Implementation	45

JuMP Model Structure	45

Validation: 3-Bus LOPF	47

Validation on the IEEE 14-Bus Standard Test Case	48

AC Power Flow on IEEE 14-Bus	49

DC Power Flow on IEEE 14-Bus	51

LOPF on IEEE 14-Bus	52

Multi-Period LOPF with Storage and Wind	53

Mathematical Extension	53

Test Scenario and Results	54

Multi-Period LOPF Performance	56

Comprehensive Component Validation	57

Performance Benchmark Results	60

DC Power Flow Results	60

AC Power Flow Results	61

LOPF Results	62

Benchmark Visualizations	62

Analysis and Discussion	63

Why DC PF Speedup Decreases with Network Size	63

Why LOPF Speedup Increases with Network Size	63

AC Power Flow: Algorithm vs. Language Overhead	64

JIT Compilation Amortization	65

Summary of Implementation Challenges	65

Network Object Model and PyPSA-Compatible API	66

Component Type Hierarchy	66

Network Container and API	67

Transformer Tap-Ratio Correction	67

Corrected Transformer Model	67

Validation on IEEE 14-Bus	68

Locational Marginal Prices	68

Implementation	69

Validation	69

Unit Commitment	70

Mathematical Formulation	70

Implementation	70

Validation	71

Unit Commitment Performance	71

AI Component: LSTM Load Forecasting and Stochastic Dispatch	72

Synthetic Training Data	73

LSTM Forecaster Training	73

Forecast Quality	74

Stochastic LOPF Results	75

Julia Technology: Flux.jl and Stochastic Solver	77

Conclusions and Future Work	78

Summary of Results	78

Conclusions	81

Future Work	83

References	86

Abstract

Power system simulation and electricity-market optimization underpin the planning and operation of modern grids, yet the dominant open-source framework for this domain, PyPSA, faces a performance limitation: for large networks and iterative scenario studies its Python-based model-assembly layer can dominate the total runtime, no self-contained Julia reimplementation with a PyPSA-compatible interface exists, and the integration of a calibrated demand forecast into a stochastic dispatch within a single native framework remains largely unexplored. This thesis reimplements PyPSA’s core computational modules in Julia and augments them with an AI-assisted forecasting layer. Five solver modules — DC power flow, AC power flow (via PowerModels.jl and Ipopt), linearized AC power flow, single- and multi-period Linear Optimal Power Flow with storage and wind generation, and Unit Commitment as a mixed-integer linear program — are implemented and validated against PyPSA on a 3-bus network and the IEEE 14-bus case; for the linear-programming methods the Julia and PyPSA results are numerically identical, and the remaining differences are explained by documented per-unit conventions. Because both implementations call the same HiGHS and Ipopt solvers, the measured speedup is isolated to the compiled model-assembly layer; benchmarks across networks from 3 to 2,000 buses show speedups ranging from about 7x for AC power flow to over 750x for large Linear Optimal Power Flow problems. An LSTM load forecaster equipped with distribution-free conformal prediction intervals supplies demand scenarios to a sample-average-approximation stochastic dispatch, which reports expected cost and conditional value-at-risk — risk measures that a deterministic formulation cannot provide. The complete implementation is released as the PowerFlowJulia package.

Keywords: power system optimization, Julia, PyPSA, optimal power flow, unit commitment, load forecasting, conformal prediction, stochastic optimization, locational marginal prices.

Introduction

Relevance of the Research

The transition of electric power systems toward low-carbon operation has turned power system simulation and electricity market optimization into a computational bottleneck rather than a modeling formality. The large-scale integration of variable renewable generation, energy storage, and sector coupling has multiplied the number of decision variables, time steps, and scenarios that a single study must evaluate. Planning a decarbonization pathway for a national grid now routinely requires solving optimization problems with millions of variables across thousands of representative hours, and repeating that solve for dozens of policy or weather scenarios.

In the academic and open-research community, PyPSA (Python for Power System Analysis) has become the de facto standard framework for this class of problems. It underpins flagship models such as PyPSA-Eur and PyPSA-Earth and supports a large body of peer-reviewed work on renewable energy systems. Its strength is a clean, high-level modeling layer built on the Python scientific stack. That same layer, however, is also the source of a practical limitation: for large networks and iterative scenario studies, the time spent building the optimization model in Python — assembling constraints through pandas and the linopy/PyOMO layer — can dominate the total runtime and reach hours per solve, well before the numerical solver itself begins work.

The Julia programming language offers a direct way to address this layer. Julia combines a high-level, dynamically typed syntax with just-in-time compilation to native code, and its JuMP modeling framework compiles the constraint-assembly step instead of interpreting it. This makes Julia a credible candidate for reimplementing the modeling layer that limits PyPSA, while delegating the actual numerical solving to the same industrial solvers (HiGHS, Ipopt) that PyPSA already uses.

At the same time, the operational use of these models increasingly depends on forecasts of uncertain quantities — most importantly electricity demand. A dispatch schedule that is optimal for a single point forecast can be far from optimal, or even infeasible, once the realized demand deviates from that forecast. This motivates coupling data-driven forecasting with the physics-based optimization, so that the optimizer is informed not only by a predicted demand profile but also by a calibrated description of its uncertainty. The combination of a high-performance Julia solver layer with an uncertainty-aware machine-learning forecast defines the “AI-assisted” framing of this work and explains its relevance to both the power systems and the scientific computing communities.

Problem Statement

Despite Julia’s well-documented performance characteristics, no actively maintained, self-contained reimplementation of PyPSA’s core computational modules currently exists, and the question of how much of PyPSA’s runtime is actually recoverable by changing the modeling layer has not been answered with reproducible measurements. Equally, the integration of a statistically validated demand forecast into a stochastic dispatch formulation within a single native framework remains largely unexplored. The present work is organized around the following concrete questions:

- Can PyPSA’s core algorithms — DC and AC power flow, linear optimal power flow, and unit commitment — be reimplemented in Julia while preserving numerical equivalence with the Python reference?

- What speedup is actually attributable to the Julia modeling layer, once both implementations are made to call the same underlying solver, and how does that speedup scale with network size and planning horizon?

- Can a neural load forecaster be equipped with distribution-free prediction intervals that provide a guaranteed coverage level suitable for dispatch optimization?

- Does propagating forecast uncertainty into a scenario-based stochastic dispatch yield decision-relevant risk measures that a deterministic formulation cannot provide?

Object and Subject of Research

The object of research is the set of computational methods used for steady-state power system simulation and for the optimization of electricity market dispatch, together with the data-driven forecasting methods that supply their demand inputs.

The subject of research is the implementation of these methods in the Julia programming language with a PyPSA-compatible interface, the quantitative comparison of their performance against the PyPSA reference implementation, and the integration of an uncertainty-aware load forecast into a stochastic dispatch formulation within that implementation.

Purpose of the Research

The purpose of this thesis is to design, implement, and validate a self-contained Julia framework that reproduces the core simulation and optimization capabilities of PyPSA with measurable performance gains, and to extend it with an AI-assisted layer that forecasts electricity demand under uncertainty and propagates that uncertainty into the dispatch optimization.

Research Tasks

To achieve this purpose, the following tasks are addressed:

- Implement and validate DC and AC power flow solvers in Julia, establishing numerical equivalence with PyPSA on standard test networks.

- Implement single-period and multi-period Linear Optimal Power Flow (LOPF) with energy storage and variable renewable generation, including the extraction of locational marginal prices from the dual solution.

- Extend the LOPF to a Unit Commitment formulation as a mixed-integer linear program with minimum up/down times and start-up costs.

- Design a reproducible benchmarking methodology and quantify the speedup of the Julia implementation over PyPSA across all solver types, a range of network sizes, and several planning horizons.

- Develop a machine-learning load forecaster, equip it with conformal prediction intervals that guarantee a prescribed marginal coverage, and integrate the resulting demand scenarios into a sample-average-approximation stochastic LOPF.

Scientific Novelty

The scientific novelty of the work consists in the following:

- A self-contained, validated Julia reimplementation of PyPSA’s core computational modules with a PyPSA-compatible construction API, requiring no Python dependency at solve time, in contrast to prior partial ports that retained Python in the workflow.

- A reproducible, multi-seed decomposition of the Julia-versus-PyPSA performance difference that attributes the speedup to the model-assembly layer rather than to the numerical solver, reported as compute-bound figures with means and standard deviations rather than headline ratios.

- The integration, within a single native framework, of a conformal-prediction-calibrated demand forecast with a scenario-based stochastic dispatch, producing distribution-free coverage guarantees and decision-relevant risk measures (expected cost and conditional value-at-risk) that are unavailable to deterministic dispatch.

Practical Significance

The resulting PowerFlowJulia framework provides a drop-in path for PyPSA users to accelerate the model-building stage of large or iterative studies without changing their numerical backend, and demonstrates a concrete pipeline in which a calibrated forecast informs risk-aware dispatch decisions. The implementation, benchmark scripts, and data pipeline are released as a reproducible package.

Thesis Structure

The thesis follows a survey–design–implement–evaluate arc.

Chapter 1 reviews the state of the art: the PyPSA framework and its modeling layer, the existing Julia power-system ecosystem, and the relevant literature on load forecasting and stochastic dispatch, and it identifies the research gap addressed here.

Chapter 2 develops the mathematical and methodological foundations of every implemented method — the DC and AC power flow models, the LOPF and unit commitment formulations, the load-forecasting model with conformal prediction, and the stochastic LOPF — together with the benchmarking protocol.

Chapter 3 presents the Julia implementation of each module, validates it against PyPSA, and reports the performance benchmarks and the results of the AI-assisted dispatch experiments.

Chapter 4 synthesizes the findings, states the conclusions regarding Julia’s suitability as a high-performance substitute for PyPSA and the value of AI-assisted dispatch, and outlines directions for future work.

Literature Review

This chapter surveys the body of work on which the thesis builds. It first examines PyPSA and the structure of its modeling layer, which is the reference framework and the source of the performance limitation under study. It then reviews the Julia scientific-computing and mathematical-optimization ecosystem and the existing Julia tools for power-system analysis, distinguishing what each provides from what this thesis sets out to deliver. The final sections cover the data-driven components: short-term load forecasting, conformal prediction for distribution-free uncertainty quantification, and stochastic optimization for power systems. The chapter closes by stating the research gap that motivates the work.

Power System Analysis and the PyPSA Framework

Optimal power flow and economic dispatch are long-standing problems in power systems engineering, with formulations that date back several decades (Wood et al. 2013). The DC power flow approximation, in which the nonlinear AC equations are linearized around a flat voltage profile, has been the workhorse for market and planning studies since its formalization (Stott 1974), because it reduces the network model to a sparse linear system while retaining the active-power flow behavior that dominates dispatch economics. The locational marginal price (LMP), which assigns a distinct energy price to each network node according to the dual of the nodal balance constraint, formalizes the economic interpretation of congestion and underpins modern electricity markets (O’Neill et al. 2005; Schweppe et al. 1988).

PyPSA (Python for Power System Analysis) (Brown et al. 2018) has become the dominant open-source framework for this class of problems in the research community. It provides a high-level, component-oriented data model — buses, lines, generators, loads, storage units, and links — and assembles linear and mixed-integer optimization problems over user-defined time series. PyPSA is the engine behind continental-scale models such as PyPSA-Eur (Hörsch et al. 2018), and it has supported a large volume of published energy-system research.

The framework’s design also exposes a performance trade-off relevant to this thesis. PyPSA constructs its optimization problems in Python, building constraints through the pandas data-handling stack and an algebraic-modeling layer before handing the assembled problem to a numerical solver such as HiGHS (Huangfu and Hall 2018) for linear and mixed-integer programs, or Ipopt (Wächter and Biegler 2006) for nonlinear ones. For small problems this construction cost is negligible, but as the number of buses, snapshots, and scenarios grows, the time spent assembling the model in interpreted Python can come to dominate the wall-clock runtime, independently of the solver. This observation — that the modeling layer, not the solver, is the recoverable cost — frames the performance question of the present work.

Julia for Scientific Computing and Optimization

Julia (Bezanson et al. 2017) was designed to resolve the “two-language problem”, in which a high-level language is used for prototyping and a low-level one for performance. Through type inference, multiple dispatch, and just-in-time compilation to native code via LLVM, Julia attains performance close to C while retaining a dynamic, high-level syntax. For the present work, the decisive feature is that performance-critical inner loops — such as the assembly of an optimization model from a component list — compile to specialized native code rather than being interpreted.

The JuMP modeling framework (Lubin et al. 2023) is the central piece of the Julia optimization ecosystem. JuMP provides an algebraic modeling language embedded in Julia and a solver-independent interface to a wide range of linear, mixed-integer, and nonlinear solvers, including the same HiGHS and Ipopt backends that PyPSA uses. Because JuMP compiles the model-building step, it has been reported to assemble large optimization problems substantially faster than interpreted Python modeling layers, which is precisely the cost component that limits PyPSA at scale. JuMP therefore makes it possible to reimplement PyPSA’s modeling layer in a compiled setting while keeping the numerical solver — and hence the achieved optimum — identical, isolating the language-level contribution to performance.

Existing Julia Tools for Power System Analysis

Several projects have addressed power-system computation in Julia, but none combines an actively maintained, self-contained implementation with a PyPSA-compatible interface. This section reviews the most relevant ones.

PSA.jl (Schlachtberger et al. 2021) is the closest existing attempt at a direct PyPSA port to Julia. Developed as a companion to PyPSA, it implements a subset of the LOPF functionality using JuMP and HiGHS. However, PSA.jl is not self-contained: the network must first be constructed in Python with PyPSA, serialized to CSV files, and only then imported into Julia for solving. This workflow preserves Python as an unavoidable dependency and therefore forfeits a substantial part of the potential performance benefit. The project has moreover been unmaintained since early 2021 and is incompatible with current Julia and JuMP releases, which renders it practically unusable for new work.

EnergyModels.jl (Hörsch et al. 2019) was conceived as a more ambitious successor that would rewrite PyPSA’s full optimization core natively in Julia, motivated by the same performance arguments pursued here. The project was nevertheless abandoned around 2019 without reaching a usable state, and no active development has followed.

PowerModels.jl (Coffrin et al. 2018), developed at Los Alamos National Laboratory, is a mature and actively maintained Julia package for steady-state power-network optimization. It provides a rigorous framework for AC and DC optimal power flow formulations and supports many solvers through JuMP. PowerModels.jl is the most technically complete tool in the Julia power-systems ecosystem, and the present thesis uses it as the solver backend for the full AC power flow module. It does not, however, replicate PyPSA’s higher-level abstractions: it offers no direct equivalent of PyPSA’s economic-dispatch interface, multi-carrier energy model, time-series optimization, storage dispatch, or capacity planning.

PowerSimulations.jl (Lara et al. 2021), developed at the National Renewable Energy Laboratory, provides production-cost modeling and economic dispatch in Julia on top of the PowerSystems.jl data model. It is actively maintained and covers part of the optimization functionality present in PyPSA. It constitutes, however, an independent architecture rather than a PyPSA port: its component abstractions, data model, and API differ substantially from PyPSA’s, so it cannot serve as a drop-in replacement for existing PyPSA-based workflows.

[ТАБЛИЦА 1.1: сравнение Julia-фреймворков — столбцы: Project | Status | PyPSA API | Self-contained | Multi-period; строки: PSA.jl (Abandoned 2021 / Partial / No / No), EnergyModels.jl (Abandoned 2019 / Partial / No / No), PowerModels.jl (Active / No / Yes / No), PowerSimulations.jl (Active / No / Yes / Yes), This thesis (Active / Partial / Yes / Partial). Подпись — стиль “Table Caption” (сверху, по центру).]

The survey shows that the two projects that most directly attempted a PyPSA port (PSA.jl, EnergyModels.jl) have been abandoned for several years, while the two active alternatives (PowerModels.jl, PowerSimulations.jl) follow independent architectures and offer no PyPSA-compatible interface. No actively maintained, self-contained Julia reimplementation of PyPSA’s core algorithms currently exists.

Short-Term Load Forecasting

Short-term load forecasting is a mature field with a literature spanning statistical, machine-learning, and, more recently, probabilistic approaches (Hong and Fan 2016). Classical baselines remain strong and are the honest reference against which any learned model must be judged. Two are used throughout this thesis: the persistence forecast, which predicts that tomorrow equals today, and the seasonal-naive forecast, which predicts that a given hour equals the same hour one week earlier, thereby capturing the dominant weekly cycle of electricity demand at no modeling cost.

Among learned models, recurrent neural networks — and in particular the Long Short-Term Memory (LSTM) architecture (Hochreiter and Schmidhuber 1997) — are widely applied to load forecasting because their gated memory can in principle capture the daily and weekly periodicities of demand. In parallel, gradient-boosted decision trees, popularized by the XGBoost implementation (Chen and Guestrin 2016), are a standard and frequently winning approach for tabular forecasting tasks: they are scale-invariant, require little preprocessing, train quickly, and exploit engineered calendar and lag features effectively. Because the two model families have complementary inductive biases — sequence memory versus feature-based partitioning — this thesis evaluates both against the naive baselines rather than committing to a single architecture. The probabilistic-forecasting literature further argues that, for operational decision-making, a calibrated description of forecast uncertainty is often more valuable than a marginal improvement in point accuracy (Hong and Fan 2016), a position that directly motivates the uncertainty-quantification component below.

Conformal Prediction

Conformal prediction provides prediction intervals with a finite-sample, distribution-free coverage guarantee: under the sole assumption that the data are exchangeable, the constructed interval contains the true value with at least a user-specified probability, regardless of the underlying forecasting model or the data distribution (Angelopoulos and Bates 2022). The split (inductive) variant computes non-conformity scores on a held-out calibration set and uses an appropriate empirical quantile of those scores to size the interval. This model-agnostic property is attractive for the present work because it allows the same calibration procedure to be applied to both the LSTM and the gradient-boosting forecaster, yielding comparable, guaranteed-coverage intervals that can be propagated into the dispatch optimization. It also separates the concern of point accuracy from the concern of coverage, which is consistent with the probabilistic-forecasting argument made above.

Stochastic Optimization in Power Systems

When demand is uncertain, a dispatch optimized for a single point forecast is generally suboptimal once the realized demand deviates from it. Stochastic programming addresses this by optimizing over a set of scenarios rather than a single realization. The Sample Average Approximation (SAA) method (Shapiro et al. 2021) approximates the expected-cost objective by an empirical average over a finite scenario sample, with the approximation improving as the number of scenarios grows; it is a standard and theoretically grounded approach for two-stage stochastic problems such as scenario-based dispatch.

Optimizing expected cost alone, however, ignores tail risk. The Conditional Value-at-Risk (CVaR) (Rockafellar and Uryasev 2000) measures the expected cost in the worst fraction of scenarios and is a coherent risk measure with a convex, optimization-friendly representation. Reporting CVaR alongside the expected cost characterizes both the average and the adverse-case behavior of a dispatch schedule under demand uncertainty — information that a deterministic formulation cannot provide. This thesis combines conformal-calibrated demand scenarios with an SAA stochastic LOPF and reports both expected cost and CVaR.

Research Gap

The reviewed literature reveals a two-part gap. First, on the computational side, no actively maintained, self-contained Julia reimplementation of PyPSA’s core modules exists: prior ports either retained Python in the workflow or were abandoned, and the active Julia tools follow non-PyPSA architectures. The extent to which PyPSA’s runtime is recoverable by replacing only the modeling layer — while keeping the numerical solver fixed — has not been quantified with a reproducible, multi-seed protocol. Second, on the data-driven side, while load forecasting, conformal prediction, and stochastic dispatch are individually well studied, their integration into a single native framework — in which a conformally calibrated forecast supplies the scenarios for a risk-aware stochastic dispatch — remains largely unaddressed. The present thesis targets both parts of this gap: a validated, self-contained, PyPSA-compatible Julia implementation with a reproducible performance comparison, extended by an AI-assisted layer that couples calibrated forecasting with stochastic, risk-aware optimization.

Design and Methodology

This chapter develops the theoretical and methodological foundations on which the remainder of the thesis rests. The first three sections derive the mathematical model underlying each algorithm: the DC power flow as a sparse linear system obtained by linearising the full AC equations, the AC power flow as a system of nonlinear nodal balance equations solved by Newton–Raphson iteration, and the Linear Optimal Power Flow as a linear programme that couples generator dispatch decisions with DC network flow constraints. The subsequent sections survey the Julia and Python software ecosystems used for implementation and highlight the key architectural differences between them. The chapter concludes by characterising the randomised network generator constructed to produce reproducible test cases of controlled size, and by specifying the JIT-aware timing methodology that ensures statistically valid performance comparisons between the two language implementations.

DC Power Flow: Mathematical Foundation

Power System Model

A power network is modeled as a graph G=(N,E), where N is the set of n buses (nodes) and E is the set of transmission lines (edges). Each bus i is characterized by its voltage Vi and phase angle θi. Each line (i,j)∈E has reactance xij and susceptance bij=1/xij (in per-unit) or bij=Vnom2/xij (in MW/rad when using physical units).

Susceptance Matrix Construction

The DC power flow approximation assumes that:

- all voltage magnitudes equal their nominal value (|Vi|≈1 p.u.),

- line resistances are negligible (rij≪xij),

- angle differences between adjacent buses are small (sinθ≈θ).

Under these assumptions, the active power injected at bus i is:

$$ P_i = {j } B{ij},_j,

$$ where B∈Rn×n is the susceptance matrix (also called the DC admittance matrix or B-matrix):

$$ B_{ij} =

$$ The diagonal entries Bii accumulate the susceptances of all lines incident to bus i, while off-diagonal entries Bij represent the negative susceptance of the line between buses i and j. The matrix B is symmetric, sparse, and singular (the vector 1 lies in its null space).

Slack Bus and Reduced System

Since the absolute voltage angle has no physical meaning, we designate bus 1 as the slack bus (or reference bus) with θ1=0. This removes the singularity by reducing the system: we partition buses into the slack bus (index 1) and the remaining n-1 buses (indices 2,…,n), and solve the reduced linear system:

$$ , = ,

$$ where B=B[2:n, 2:n] is the reduced susceptance matrix (always non-singular for connected networks), θ=(θ2,…,θn)⊤ is the vector of unknown angles, and P=(P2,…,Pn)⊤ is the vector of net power injections (generation minus load) at all non-slack buses.

Active Power Flows

Once θ is obtained, the active power flow on line (i,j) from bus i to bus j is:

$$ p_{ij} = b_{ij},(_i - _j).

$$ ### Algorithm Design

The Julia implementation follows four steps:

- Assemble the n×n susceptance matrix B from the line list.

- Compute the net injection vector P=Pgen-Pload.

- Solve the reduced system (eq.) using Julia’s built-in backslash operator (which dispatches to LAPACK for dense or SuiteSparse for sparse matrices).

- Compute per-line flows via Equation (eq.).

For large networks (e.g. n>500), the susceptance matrix is stored as a sparse matrix to exploit the sparsity of real transmission networks. Typical transmission grids have |E|∼1.5 n, resulting in a matrix density below 1%.

AC Power Flow: Mathematical Foundation

Full AC Power Flow Equations

The AC power flow model represents the full nonlinear relationships between complex voltages and power injections. At each bus i, the complex voltage is Vi=|Vi|ejθi. The complex power injection is Si=Pi+jQi, where Pi is active power and Qi is reactive power.

The nodal power balance equations are derived from the admittance matrix Ybus:

$$$$ where θij=θi-θj, and Gij+jBij is the (i,j) entry of Ybus.

Newton-Raphson Solution Method

The AC power flow is solved iteratively using the Newton-Raphson method. Define the mismatch vector:

$$ () =

= ,

$$ where x=(θ⊤, |V|⊤)⊤ is the state vector. The Newton-Raphson update at iteration k is:

$$ {(k)},{(k)} = (^{(k)}),

\qquad

\mathbf{x}^{(k+1)} = \mathbf{x}^{(k)} + \Delta\mathbf{x}^{(k)},

$$ where J=∂f/∂x is the Jacobian matrix, which has the block structure:

$$  =

.

$$ Convergence is declared when ∥f(x(k))∥∞<ε (typically ε=10-6 p.u.).

Per-Unit System

AC power flow computations use the per-unit (p.u.) system to normalize quantities and improve numerical conditioning. For a system with base apparent power Sbase=100 MVA and base voltage Vbase:

$$ P_ = ,

Z_\text{p.u.} = \frac{Z_\Omega \cdot S_\text{base}}{V_\text{base}^2}.

$$ All generator outputs, loads, and line impedances must be converted to per-unit before solving. The Julia implementation performs this conversion explicitly, a step that was a key source of errors during development.

PowerModels.jl Integration

For the AC power flow implementation, this thesis uses PowerModels.jl (Coffrin et al. 2018), a Julia package that provides a general framework for formulating and solving power network optimization problems. PowerModels.jl uses a standardized data dictionary format (inspired by Matpower (Zimmerman et al. 2011)) and interfaces with Ipopt (Wächter and Biegler 2006) for nonlinear optimization.

The AC power flow is cast as a nonlinear optimization problem (minimizing generation cost subject to power flow constraints), which Ipopt solves via interior-point methods.

Linearized AC Power Flow: Mathematical Foundation

Motivation and Position in the Hierarchy

The DC power flow (Section 2.1) and full AC power flow (Section 2.2) represent two extremes: the DC approximation is computationally trivial but ignores resistance, reactive power, and voltage magnitude variations; the full Newton–Raphson AC solution is exact but requires iterative nonlinear solves. The Linearized AC Power Flow (LACPF) occupies the middle ground: it retains the computational simplicity of a single linear solve while recovering both voltage magnitude deviations and reactive power flows that the DC approximation discards.

First-Order Taylor Linearisation

The LACPF is derived by applying a first-order Taylor expansion to the full AC power flow equations around the flat start operating point, where all voltage magnitudes equal 1 p.u. and all angles equal zero. Let Δθ=θ-0 and Δ|V|=|V|-1 denote deviations from the flat start. The linearised nodal power balance is:

$$

\begin{pmatrix} \Delta\boldsymbol{\theta} \\ \Delta|\mathbf{V}| \end{pmatrix}

=

\begin{pmatrix} \mathbf{P} \\ \mathbf{Q} \end{pmatrix},

$$ where B'∈Rn×n and G'∈Rn×n are the susceptance Laplacian and conductance Laplacian of the network graph, respectively:

$$$$ with branch susceptance bij=xij/(rij2+xij2) and conductance gij=rij/(rij2+xij2) in per-unit. The slack bus is removed by reducing the system to 2(n-1)×2(n-1), analogously to the DC power flow reduction.

Comparison with DC Power Flow

Setting G'=0 (lossless lines, r=0) and ignoring the Q equation reduces Equation (eq.) to the standard DC power flow equation B'Δθ=P. The LACPF therefore strictly extends the DC approximation by:

- Including line resistance through G', improving active power flow accuracy.

- Solving for voltage magnitude deviations Δ|V| that the DC model cannot provide.

- Providing first-order estimates of reactive power flows Q.

However, reactive power estimates from the LACPF are known to have larger errors than active power estimates, because reactive power is more sensitive to second-order voltage effects that are discarded in the linearisation (Stott 1974).

Linear Optimal Power Flow: Mathematical Foundation

Problem Formulation

The Linear Optimal Power Flow (LOPF) combines the DC power flow approximation with economic dispatch optimization. The goal is to dispatch generators to minimize total generation cost while satisfying network constraints. The LOPF is a linear program (LP):

$$ {^,, } {i } c_i,P_i^

$$ subject to:

$$$$ where G⊆N is the set of generator buses, ci is the marginal cost of generator i (€/MWh), Fijmax is the thermal limit of line (i,j), and Pimax is the installed generator capacity.

Connection to DC Power Flow

Equation (eq.) is precisely the DC power flow balance (eq.), with the generator output Pigen as an additional decision variable. The LOPF thus extends DC power flow by making generator dispatch optimal rather than fixed. The voltage angles θ serve as intermediate variables linking generation and line flows.

LP Solver Interface Design via JuMP

The Julia implementation uses JuMP.jl (Lubin et al. 2023), a domain-specific modeling language embedded in Julia, to declare the optimization model. JuMP separates the model description from the solver, enabling transparent solver switching. The workflow is:

- Declare a Model object with the HiGHS solver backend.

- Register decision variables (@variable) with bounds.

- Add constraints (@constraint) for power balance and line limits.

- Set objective (@objective) to minimize marginal cost.

- Call optimize!(model) and extract results.

HiGHS (Huangfu and Hall 2018) is used as the LP solver for both the Julia (via HiGHS.jl) and Python (via PyPSA/linopy) implementations, ensuring a fair comparison of language overhead rather than solver differences.

Technology Stack and Tool Selection

Julia Ecosystem

The Julia implementation uses the following packages, all available through Julia’s built-in package manager (Pkg):

Package Version Purpose

LinearAlgebra stdlib Dense matrix operations, backslash solver

SparseArrays stdlib Sparse matrix construction and operations

JuMP 1.29 Optimization modeling language

HiGHS 1.21 LP/MIP solver

PowerModels 0.21 AC power flow, network data format

Ipopt 1.14 Nonlinear interior-point solver

: Julia packages used in the implementation

Python Ecosystem

The Python reference implementation uses PyPSA (Brown et al. 2018), which internally relies on linopy for LP/MILP modeling and HiGHS for LP solving.

Package Version Purpose

PyPSA 0.28+ Power system analysis framework

NumPy 1.26+ Array operations

linopy 0.3+ LP modeling (used internally by PyPSA)

HiGHS 1.7+ LP solver backend

: Python packages used as reference implementation

Comparative Analysis of Ecosystems

Table (ref.) summarizes the key differences between the Julia and Python ecosystems relevant to this thesis.

Aspect Julia Python

Execution model JIT-compiled (LLVM) Interpreted

Linear algebra Direct LAPACK calls NumPy (LAPACK + overhead)

LP modeling JuMP (native) linopy/Pyomo (Python objects)

Startup time High (TTFX) Low

Matrix operations Native performance C extension calls

Type system Multiple dispatch Dynamic typing

Sparse matrices Native (stdlib) scipy.sparse

: Comparison of Julia and Python ecosystems for power system computation

A critical difference is Julia’s time to first execution (TTFX): the first call to any function triggers JIT compilation, which can take several seconds. However, subsequent calls run at native speed. The benchmarks in this thesis exclude warmup time to measure steady-state performance.

Benchmark Methodology

Random Network Generator

To enable reproducible, fair comparisons between Julia and Python implementations, a random network generator was implemented using the same algorithm and the same random seed (seed=42) in both languages. The generator produces a connected network by:

- Constructing a spanning tree: bus i connects to bus i+1 with reactance x∼U(0.05, 0.50) p.u., ensuring connectivity.

- Adding approximately ⌊n/3⌋ additional random edges to create mesh topology.

- Assigning random loads Pload∼U(50, 500) MW to 70% of non-slack buses.

- Placing generators at bus 1 (slack, capacity = 1.1×∑Pload) and at every 4th bus.

Networks are generated for sizes n∈{3,10,50,100,500,1000,2000} buses for DC power flow, and n∈{3,10,50,100,500} buses for LOPF (larger sizes were omitted due to Python runtime constraints).

JIT Warmup Strategy

Julia’s JIT compiler compiles functions on first call. To measure steady-state (post-compilation) performance, each benchmark includes a mandatory warmup call:

- Call the function once to trigger JIT compilation.

- Run the function N times and record execution times.

- Report the median of the N recorded times.

The median is preferred over the mean because it is robust to occasional outliers caused by garbage collection or OS scheduling jitter. The number of runs N scales inversely with network size: N=200 for n≤100, N=50 for n≤500, and N=10 for n>500 in DC power flow; analogously for LOPF.

Python Timing Methodology

For the Python benchmark, network construction is separated from the solver call. For DC power flow (PyPSA’s lpf()), the Network object is built once and lpf() is called repeatedly. For LOPF (PyPSA’s optimize()), the Network is rebuilt on every run, because PyPSA modifies its internal state during optimization, which would make repeated calls measure incremental rather than full solve time.

Hardware and Software Environment

All benchmarks were executed on a single machine under identical conditions. The operating system is Windows 11 Pro (build 26100). Julia version 1.12 and Python 3.11 were used. No parallel computation or GPU acceleration was employed; all computations are single-threaded to ensure a direct comparison of sequential algorithmic performance.

Unit Commitment: Mathematical Foundation

Unit Commitment (UC) extends the multi-period LOPF by adding binary on/off decisions for generators, minimum operating time constraints, and fixed startup and shutdown costs. The resulting problem is a Mixed-Integer Linear Program (MILP). For each committable generator g and time period t∈{1,…,T}, three binary variables are defined: ug,t∈{0,1} (commitment status), σg,t+∈{0,1} (startup event), and σg,t-∈{0,1} (shutdown event), linked by the transition constraint:

$$ u_{g,t} - u_{g,t-1} = {g,t}^+ - {g,t}^-, g,t.

$$ The dispatch bounds couple binary and continuous variables:

$$ p_g^,u_{g,t} ;; P_{g,t} ;; p_g^,u_{g,t}.

$$ Minimum up-time (Tgup) and down-time (Tgdn) are enforced by the rolling-window constraints:

$$$$ The extended objective minimises dispatch cost plus fixed startup and shutdown costs:

$$ _{g,t} !

$$ subject to the power balance (eq.) and storage dynamics (eq.)–(eq.) from Section 2.4. Because UC contains integer variables, Locational Marginal Prices cannot be extracted directly from the MILP dual; instead, the binary variables are fixed at their optimal values and a continuous LP relaxation is re-solved to recover price-consistent LMPs (O’Neill et al. 2005).

AI Component: LSTM Load Forecasting

Motivation

Demand uncertainty is a fundamental challenge in power system operation. Even modest forecast errors — on the order of 5–15% Mean Absolute Percentage Error (MAPE) — propagate into suboptimal dispatch, unnecessary reserve activations, and higher operational costs. The AI component of this thesis introduces a data-driven LSTM forecaster that produces not only a point forecast of the next 24-hour load profile, but also statistically valid prediction intervals, which are then fed into a stochastic dispatch optimization.

LSTM Architecture

A Long Short-Term Memory (LSTM) network (Hochreiter and Schmidhuber 1997) is used for its ability to capture temporal dependencies in load time series. The cell state dynamics at each time step t are:

$$$$ where σ(⋅) is the sigmoid function and ⊙ denotes element-wise multiplication. The hidden state ht at the last input step is passed through a linear (Dense) layer to produce a 24-hour output vector.

The architecture used in this thesis is: input dimension 1 (scalar hourly load in p.u.), hidden size 32, output size 24 (horizon). The model is implemented in Flux.jl (Innes 2018) and trained with the Adam optimiser (Kingma and Ba 2014) (learning rate 10-3) with early stopping (patience 10 epochs on a held-out validation set of 15% of the training data).

Training Protocol

The input data is structured as a sliding-window dataset: each sample consists of a 24-hour input window x=(zt,zt+1,…,zt+23) and a 24-hour target y=(zt+24,…,zt+47), where zt=(xt-μ)/σ is the z-score normalised load. The dataset is split into training (75%), validation (15%), and calibration (10%) subsets in chronological order. Model parameters are restored from the epoch with lowest validation RMSE.

Conformal Prediction Intervals

Standard neural network confidence intervals are not statistically guaranteed. This thesis instead applies split conformal prediction (Angelopoulos and Bates 2022) to produce intervals with a distribution-free coverage guarantee.

The non-conformity score for a calibration sample (xi,yi) is the maximum absolute prediction error over the 24-hour horizon:

$$ s_i = {j=1}^{24} |{ij} - y_{ij}|, i = 1, , n_.

$$ The conformal quantile at coverage level 1-α is:

$$  = s_{(1-)(n_+1) },

$$ where scores are sorted in ascending order. The prediction interval for a new input x* is:

$$ [_j^* - ,; _j^* + ], j = 1,,24.

$$ By Theorem 1 of (Angelopoulos and Bates 2022), this interval satisfies P(yj*∈[yj*-q,yj*+q])≥1-α for exchangeable data, without any distributional assumptions on the residuals.

Stochastic LOPF via Sample Average Approximation

Uncertainty Model

The conformal prediction intervals define a set of plausible 24-hour load profiles. To quantify the resulting cost uncertainty, S scenario profiles are sampled from N(y*,σj2) clipped to [yj*-q, yj*+q], where σj=(q)/2 is chosen so that approximately 95% of the Gaussian mass lies within the conformal interval.

Sample Average Approximation

The Sample Average Approximation (SAA) (Shapiro et al. 2021) replaces the true expectation E[f(x,ξ)] with an empirical average:

$$ {x}  {s=1}^{S} f(x, _s),

$$ where ξs is the s-th load scenario vector. In this thesis, the SAA is implemented by solving S independent multi-period LOPFs and aggregating the results:

$$ []  _{s=1}^{S} C^*(s),

C^*(s) = \text{LOPF cost under scenario } s.

$$ ### Conditional Value-at-Risk

Beyond the expected cost, the α-CVaR quantifies tail risk — the expected cost in the worst (1-α) fraction of scenarios (Rockafellar and Uryasev 2000):

$$ {1-} =  {s } C^*(s),

$$ where W is the index set of the ⌊(1-α)S⌋ highest-cost scenarios. In this thesis, α=0.90 (CVaR​90) is used, corresponding to the expected cost in the worst 10% of scenarios. CVaR is a coherent risk measure (Rockafellar and Uryasev 2000) and is preferred over Value-at-Risk because it captures the full tail distribution rather than a single quantile.

Implementation and Experiments

This chapter presents the Julia implementations of the three power flow modules alongside their numerical validation and performance evaluation. The chapter opens with the definition of a canonical 3-bus test network that serves as a common validation fixture throughout. The DC power flow, AC power flow, and LOPF implementations are then described in turn: each section presents the core source code, discusses the engineering decisions and non-obvious challenges encountered during porting, and reports numerical results compared against the reference PyPSA implementation. The chapter concludes with a systematic performance benchmark spanning networks from 3 to 2,000 buses, followed by an analysis that accounts for the observed speedup patterns — including the opposing trends exhibited by DC power flow and LOPF as problem size grows.

Test Network Definition

All three implementations are validated on the same standard 3-bus test network, ensuring consistency across comparisons with PyPSA. The network is defined as follows:

- Bus 1: Generator G1, capacity P1max=400 MW, marginal cost c1=20 EUR/MWh, reference bus (θ1=0).

- Bus 2: Generator G2, capacity P2max=300 MW, marginal cost c2=50 EUR/MWh; load L2=200 MW.

- Bus 3: Load L3=300 MW, no local generation.

All three transmission lines have reactance x=0.1 p.u. (base: Sbase=100 MVA, Vbase=380 kV), giving a susceptance of:

$$ b =  =  = 1000,.

$$ The susceptance matrix of this network is:

$$  =

.

$$ ## DC Power Flow Implementation

Core Implementation

The DC power flow solver is implemented in julia/dc_power_flow.jl. The core function assembles the susceptance matrix and solves the linear system using Julia’s backslash operator, which dispatches to LAPACK for dense matrices:

function dc_pf_solve(n_buses, lines, generators, loads; baseMVA=100.0)

    # Build susceptance matrix B (n x n)

    B = zeros(n_buses, n_buses)

    for (from, to, x) in lines

        b = baseMVA / x          # b [MW/rad] = S_base / x_pu

        B[from, from] += b

        B[to,   to  ] += b

        B[from, to  ] -= b

        B[to,   from] -= b

    end



    # Net injection vector P = P_gen - P_load

    P = zeros(n_buses)

    for (bus, p) in generators;  P[bus] += p;  end

    for (bus, p) in loads;       P[bus] -= p;  end



    # Remove slack bus (bus 1): solve reduced (n-1) x (n-1) system

    idx   = 2:n_buses

    theta = zeros(n_buses)

    theta[idx] = B[idx, idx] \ P[idx]    # Julia backslash -> LAPACK

    return theta

end

Validation

For the DC power flow validation, the generator dispatch is fixed at G1=400 MW (slack, absorbs imbalance), G2=100 MW (the economically optimal dispatch from LOPF Scenario A). The net injections are:

$$  =

=

,.

$$ Solving the reduced 2×2 system Bθ=P analytically:

$$

\begin{pmatrix} \theta_2 \\ \theta_3 \end{pmatrix}

= \begin{pmatrix} -100 \\ -300 \end{pmatrix}

\implies

\begin{pmatrix} \theta_2 \\ \theta_3 \end{pmatrix}

= \begin{pmatrix} -1/6 \\ -7/30 \end{pmatrix}\,\text{rad}.

$$ Table (ref.) compares the Julia and PyPSA results on this network.

Quantity Julia PyPSA Relationship

θ2 (rad) -0.1667 -1.15×10-4 ratio =1444

θ3 (rad) -0.2333 -1.62×10-4 ratio =1444

p12 (MW) 166.7 166.7 Δ<10-6

p13 (MW) 233.3 233.3 Δ<10-6

p23 (MW) 66.7 66.7 Δ<10-6

: DC power flow validation: Julia vs PyPSA (3-bus network, G1=400 MW, G2=100 MW). Branch flows agree exactly; voltage angles differ by a constant per-unit convention factor of Vnom2/Sbase=3802/100=1444 (see text).

The branch flows — the physically and economically meaningful output of the DC power flow — agree to better than 10-6 MW. The voltage angles differ by an exact constant factor of 1444 because the two implementations adopt different per-unit conventions for the line reactance. PyPSA interprets the Line.x attribute as an ohmic value and converts it internally to per-unit by dividing by Vnom2/Sbase=3802/100=1444 Ω, producing angles of order 10-4 rad. The present implementation, following the MATPOWER convention, treats x as already in per-unit and forms the susceptance as b=Sbase/x in MW/rad, producing angles of order 10-1 rad. Because the angle scale cancels exactly in the branch-flow expression pkm=bkm(θk-θm), the resulting flows are identical. The difference is therefore a documented input-convention choice, not a modelling error; the same flows, dispatch, and prices are obtained under both conventions. Notably, this scale difference appears only in the synthetic toy network, where a bare per-unit reactance (x=0.1) is supplied against a 380 kV base; on the IEEE 14-bus case of Section (ref.), where the MATPOWER data carry physically consistent per-unit reactances, the Julia and PyPSA angles agree to better than 0.001∘ across all buses (Table (ref.)). Figure (ref.) shows the voltage angles at each bus.

Validation results on the 3-bus test network (left to right):

- DC Power Flow voltage angles at each bus — Bus 1 is the reference (θ1 = 0∘), with angles of −9.55∘ and −13.37∘ at Buses 2 and 3;

- LOPF generator dispatch for Scenario A (unconstrained, blue) and Scenario B (200 MW thermal limit, red);

- transmission line power flows for both scenarios, with the 200 MW thermal limit shown as a dashed line — Line 1-3 is at 116.7% loading in Scenario A and exactly at the limit in Scenario B.

AC Power Flow Implementation

PowerModels.jl Data Format

The AC power flow is implemented using PowerModels.jl, which requires a specific data dictionary format. Building this format from scratch revealed several non-obvious requirements:

- Per-unit conversion: All quantities must be in p.u. (Sbase=100 MVA). Generator output, load power, and line impedances must all be divided by their respective base values. Failing to convert leads to angles of hundreds or thousands of radians in the solver output.

- Mandatory shunt admittance fields: Each branch requires explicit g_fr, b_fr, g_to, b_to fields (shunt conductance and susceptance at the from- and to-ends of the line). These are not auto-computed; for lossless lines, g_fr = g_to = 0 and b_fr = b_to =bc/2, where bc is the line charging susceptance. This requirement was discovered by reading the PowerModels.jl source (matpower.jl:454).

- Angle bounds in radians: The angmin/angmax fields must be in radians (±π/3≈±60∘). Supplying values in degrees makes the bounds effectively infinite.

- Branch flow extraction: The build_pf formulation uses JuMP affine expressions (not variables) for branch flows, so they do not appear in the solution dictionary. Branch flows must be computed post-solution via the helper calc_branch_flow_ac from PowerModels.

Implementation

The key function converting the network to PowerModels format is shown below (condensed for clarity):

function network_to_powermodels(buses, lines, generators, loads,

                                 v_nom=380.0; baseMVA=100.0)

    data = Dict{String,Any}(

        "baseMVA"  => baseMVA,

        "per_unit" => true,

        "shunt"    => Dict{String,Any}(),   # required empty dicts

        "dcline"   => Dict{String,Any}(),

        "storage"  => Dict{String,Any}(),

        "switch"   => Dict{String,Any}(), ...

    )

    # Branch: convert to per-unit + add shunt admittance fields

    Z_base = v_nom^2 / baseMVA          # base impedance [Ohm]

    br_r = r / Z_base;  br_x = x / Z_base

    br_b = 1.0 / sqrt(br_r^2 + br_x^2) * 0.1  # ~10% charging

    branch_data = Dict(

        "br_r"  => br_r,  "br_x"  => br_x,

        "g_fr"  => 0.0,   "b_fr"  => br_b/2.0,   # required!

        "g_to"  => 0.0,   "b_to"  => br_b/2.0,   # required!

        "angmin" => -pi/3, "angmax" => pi/3, ...)

    # Generator: per-unit active power

    gen_data = Dict("pg" => P_MW / baseMVA,

                    "pmax" => P_max / baseMVA, ...)

end

Branch Flow Extraction

After solving, branch flows are recovered in two steps: the solution is merged back into the data dictionary, and then the branch flows are computed from the recovered voltages:

PowerModels.update_data!(pm_data, result["solution"])

branch_flows = PowerModels.calc_branch_flow_ac(pm_data)

for (i, branch) in sort(branch_flows["branch"])

    pf_MW = branch["pf"] * baseMVA     # convert from p.u. to MW

    pt_MW = branch["pt"] * baseMVA

end

Validation

The AC power flow implementation is validated on the same 3-bus test network defined in Section 3.1, with the generator dispatch fixed at G1=500 MW (slack), loads L2=300 MW and L3=200 MW, and line parameters r=0.01 p.u., x=0.1 p.u. (Sbase=100 MVA, Vbase=380 kV).

Table (ref.) compares the Julia (PowerModels.jl + Ipopt) and PyPSA (network.pf()) results.

Quantity Julia PyPSA Max |Δ|

|V1| (p.u.) 1.000000 1.000000 0

|V2| (p.u.) 0.999982 0.999982 4.8×10-7

|V3| (p.u.) 0.999984 0.999984 4.8×10-7

θ1 (rad) 0.000000 0.000000 0

θ2 (rad) -0.000185 -0.000185 4.1×10-7

θ3 (rad) -0.000162 -0.000162 4.1×10-7

p12 (MW) 266.67 266.67 3.5×10-3

p13 (MW) 233.34 233.34 3.5×10-3

p23 (MW) -33.33 -33.33 3.5×10-3

: AC power flow validation: Julia vs PyPSA (3-bus network)

All quantities agree with PyPSA to better than 5×10-3 absolute error. The small residual in line active power (3.5×10-3 MW) reflects the difference between Ipopt’s interior-point convergence tolerance (10-8 p.u.) and PyPSA’s Newton–Raphson tolerance (10-6 p.u.), not a modelling error. Voltage magnitudes and angles agree to <5×10-7, confirming that the per-unit conversion, admittance matrix construction, and PowerModels.jl data format are all correct.

Linearized AC Power Flow Implementation

Implementation

The Linearized AC Power Flow is implemented in julia/solvers/linear_ac_pf.jl as a direct linear solve of Equation (eq.). The key difference from the DC power flow is the construction of both the susceptance Laplacian B' and the conductance Laplacian G', and the assembly of the 2(n-1)×2(n-1) coupled system:

z_base = v_nom^2 / baseMVA          # 380^2 / 100 = 1444 Ohm

for (from, to, r, x) in lines

    r_pu = r / z_base;  x_pu = x / z_base

    denom = r_pu^2 + x_pu^2

    g_ij = r_pu / denom;  b_ij = x_pu / denom

    # Susceptance Laplacian B'

    B[from,from] += b_ij;  B[to,to] += b_ij

    B[from,to]   -= b_ij;  B[to,from] -= b_ij

    # Conductance Laplacian G'

    G[from,from] += g_ij;  G[to,to] += g_ij

    G[from,to]   -= g_ij;  G[to,from] -= g_ij

end

# 2(n-1) x 2(n-1) coupled system

A = [ B_r  G_r ; -G_r  -B_r ]

x_sol = A \ [P_r; Q_r]       # single linear solve

dtheta, dv = x_sol[1:nm], x_sol[nm+1:end]

V_mag = 1.0 .+ dv;  theta = dtheta

The implementation requires no iterative solver — a single backslash call (dispatching to LAPACK) solves the coupled active/reactive system. The computational cost is O((2n)3) for dense matrices, but the sparsity of real networks means sparse LU factorisation is used for large n.

Validation

Table (ref.) compares the LACPF solution against the full AC power flow reference (PyPSA Newton–Raphson) and the DC power flow, on the standard 3-bus network.

Quantity LACPF Full AC (ref) DC PF |Δ| LACPF

θ2 (rad) -0.000185 -0.000185 -0.000185 3.3×10-7

θ3 (rad) -0.000162 -0.000162 -0.000162 4.1×10-7

|V2| (p.u.) 1.000018 0.999982 1.000000 3.7×10-5

|V3| (p.u.) 1.000016 0.999984 1.000000 3.2×10-5

p12 (MW) 261.39 266.67 266.67 5.28 MW

q12 (MVAr) -52.81 0.048 N/A 52.85 MVAr

: Linearized AC PF validation: LACPF vs full AC PF reference (3-bus network)

Voltage angles and magnitudes agree with the full AC reference to <4×10-5 p.u. and <5×10-7 rad, respectively — a substantial improvement over the DC approximation, which cannot estimate voltage magnitudes at all.

Active power flows show a 5.28 MW discrepancy (≈2%) compared to the full AC solution. This difference arises because the LACPF includes the resistive correction through G': in a lossless DC network the flow is 266.67 MW, while the LACPF redistributes ≈5 MW into resistive losses.

Reactive power flows exhibit a large error (52.85 MVAr). This is expected: the flat-start linearisation assumes Q(0)=0, but the actual reactive power depends on second-order voltage effects that are entirely discarded at first order. For applications requiring accurate reactive power, the LACPF serves as a warm start for Newton–Raphson, not a replacement.

LOPF Implementation

JuMP Model Structure

The LOPF is implemented in julia/lopf.jl using JuMP.jl. The model formulation maps directly to Equations (eq.)–(eq.):

function solve_lopf(buses, lines, generators, loads, line_names;

                    line_capacity=Inf, baseMVA=100.0)

    n_buses  = length(buses)

    gen_buses = sort(collect(keys(generators)))



    # Susceptance matrix [MW/rad]

    B = zeros(n_buses, n_buses)

    sus = Float64[]

    for (from, to, r, x) in lines

        b = baseMVA / x          # x in per-unit

        push!(sus, b)

        B[from,from]+=b; B[to,to]  +=b

        B[from,to]  -=b; B[to,from]-=b

    end



    model = Model(HiGHS.Optimizer);  set_silent(model)



    @variable(model, theta[1:n_buses])

    @variable(model, P_gen[bus in gen_buses],

              lower_bound=0.0,

              upper_bound=generators[bus][1])    # capacity



    @constraint(model, theta[gen_buses[1]] == 0.0)   # slack

    for k in 1:n_buses

        P_inj = (k in gen_buses) ? P_gen[k] : 0.0

        @constraint(model,

            sum(B[k,m]*theta[m] for m in 1:n_buses) == P_inj - P_load[k])

    end

    if isfinite(line_capacity)

        for (i,(from,to,_,_)) in enumerate(lines)

            f = sus[i] * (theta[from] - theta[to])

            @constraint(model, -line_capacity <= f <= line_capacity)

        end

    end

    @objective(model, Min,

        sum(generators[bus][2] * P_gen[bus] for bus in gen_buses))

    optimize!(model)

end

Validation: 3-Bus LOPF

Using the test network defined in Section 3.2, the LOPF was solved under two scenarios:

- Scenario A (unconstrained): No line capacity limits (Fijmax=∞).

- Scenario B (congested): Line thermal limit Fijmax=200 MW on all lines.

Scenario P1 (MW) P2 (MW) Max flow (MW) Cost (€/h) Matches?

A: No limits 400 100 233 13,000 Yes (Δ=0)

B: 200 MW limit 300 200 200 16,000 Yes (Δ=0)

: LOPF validation: 3-bus network, two constraint scenarios (Julia vs PyPSA)

Scenario A (unconstrained): The optimizer dispatches the cheap generator G1 (c1=20 EUR/MWh) as much as possible and uses G2 (c2=50 EUR/MWh) only to cover the residual:

$$ P_1 = 400,,P_2 = 100,,

\text{Cost} = 400 \times 20 + 100 \times 50 = 13{,}000\,\text{EUR/h}.

$$ The maximum line flow is p13=233 MW (line 1–3), which exceeds the 200 MW thermal limit.

Scenario B (200 MW limit): The binding constraint on line 1–3 forces a different dispatch. G1 is reduced and G2 must compensate:

$$ P_1 = 300,,P_2 = 200,,

\text{Cost} = 300 \times 20 + 200 \times 50 = 16{,}000\,\text{EUR/h} \quad (+23.1\%).

$$ The 23.1% cost increase directly quantifies the economic cost of network congestion. Both results match PyPSA to numerical precision (Δ=0 MW, Δ=0 EUR/h).

The dispatch and line flow results for both scenarios are shown in panels (b) and (c) of Figure (ref.).

Validation on the IEEE 14-Bus Standard Test Case

The three-bus network used in Sections 3.2–3.4 is intentionally simple and designed to allow analytical verification. To demonstrate that the implementations generalise to a recognised industry benchmark, all three modules were additionally validated on the IEEE 14-bus test case (Zimmerman et al. 2011) — a standard network used throughout the power systems literature for algorithm verification and comparison.

The IEEE 14-bus network comprises 14 buses, 20 branches (including 4 transformers with off-nominal tap ratios), 5 generators, and 11 load buses with a total active demand of 259 MW (Sbase=100 MVA). The MATPOWER case file case14.m was used as the common data source for both Julia and Python, ensuring that both implementations operate on identical network data.

AC Power Flow on IEEE 14-Bus

Table (ref.) compares the Julia (PowerModels.jl + Ipopt) AC power flow solution against the reference solution stored in case14.m, which represents the MATPOWER-solved steady-state. PowerModels.jl reads and models the transformers with their correct tap ratios; the Julia implementation therefore produces a full transformer-aware solution.

+———–+———-+———-+——————————+—————————–+——————————+

Bus | |V| | |V| | |ΔV| | θ Julia (°) | |Δθ| (°) |

| Julia | Ref** | | | |

| (p.u.)** | | | | |

+==========:+=========:+=========:+=============================:+============================:+=============================:+

1 | 1.0600 | 1.0600 | 0 | 0.0000 | 0 |

+———–+———-+———-+——————————+—————————–+——————————+

2 | 1.0450 | 1.0450 | 0 | -4.9826 | 2.6×10-3 |

+———–+———-+———-+——————————+—————————–+——————————+

3 | 1.0100 | 1.0100 | 0 | -12.7251 | 5.1×10-3 |

+———–+———-+———-+——————————+—————————–+——————————+

4 | 1.0177 | 1.0190 | 1.3×10-3 | -10.3129 | 1.7×10-2 |

+———–+———-+———-+——————————+—————————–+——————————+

6 | 1.0700 | 1.0700 | 0 | -14.2209 | 9.5×10-4 |

+———–+———-+———-+——————————+—————————–+——————————+

9 | 1.0559 | 1.0560 | 6.8×10-5 | -14.9385 | 1.5×10-3 |

+———–+———-+———-+——————————+—————————–+——————————+

14 | 1.0355 | 1.0360 | 4.7×10-4 | -16.0336 | 6.4×10-3 |

+———–+———-+———-+——————————+—————————–+——————————+

Max over all 14 buses | 1.33×10-3 | | 1.71×10-2 |

+———————————+——————————+—————————–+——————————+

: IEEE 14-bus AC power flow: Julia vs MATPOWER reference

The maximum voltage magnitude error is 1.33×10-3 p.u. and the maximum angle error is 0.017∘, both well within engineering accuracy (<0.5%). The small residual reflects the difference in convergence tolerances between Ipopt and the reference solver, not a modelling error. The result confirms that the PowerModels.jl-based AC power flow implementation handles transformer tap ratios correctly on a realistic multi-machine network.

DC Power Flow on IEEE 14-Bus

Table (ref.) reports the DC power flow voltage angles from Julia and PyPSA, both compared against the full AC reference.

+————-+—————————-+—————————-+——————–+

Bus | θ Julia (°) | θ PyPSA (°) | **|Δθ| |

| | | vs AC ref (°)** |

+============:+===========================:+===========================:+===================:+

1 | 0.000 | 0.000 | 0.000 |

+————-+—————————-+—————————-+——————–+

2 | -5.013 | -5.013 | 0.033 |

+————-+—————————-+—————————-+——————–+

5 | -9.089 | -9.089 | 0.309 |

+————-+—————————-+—————————-+——————–+

6 | -15.165 | -15.165 | 0.945 |

+————-+—————————-+—————————-+——————–+

9 | -15.893 | -15.893 | 0.953 |

+————-+—————————-+—————————-+——————–+

14 | -17.429 | -17.429 | 1.389 |

+————-+—————————-+—————————-+——————–+

Max (all 14 buses) | identical | 1.389° |

+——————————————+—————————-+——————–+

: IEEE 14-bus DC power flow: Julia and PyPSA vs full-AC reference (MATPOWER)

Julia and PyPSA produce identical DC power flow angles across all 14 buses. The deviation from the full AC reference (max 1.39∘) is larger than for pure transmission networks because IEEE 14-bus contains four transformers with off-nominal tap ratios (0.932–0.978), which the DC linearisation ignores. This is a known limitation of the DC approximation, not a software defect.

LOPF on IEEE 14-Bus

Both implementations minimise total generation cost over the five generators at buses 1, 2, 3, 6, and 8. The two cheapest generators (buses 1 and 2, marginal cost 20 EUR/MWh) are preferred over the more expensive generators (buses 3, 6, 8 at 40 EUR/MWh).

Metric Julia PyPSA Match?

Total load (MW) 259.0 259.0 Yes

Total cost (€/h) 5,180 5,180 Yes

Generators 3, 6, 8 0 MW each 0 MW each Yes

: IEEE 14-bus LOPF: Julia (JuMP+HiGHS) vs PyPSA (linopy+HiGHS)

Both solvers find the same optimal objective value of 5,180 EUR/h and correctly set the three expensive generators to zero. The LP is degenerate (generators 1 and 2 share the same marginal cost of 20 EUR/MWh), so the individual dispatch at buses 1 and 2 may differ between solvers while the total cost and expensive-generator dispatch remain identical — a well-known property of degenerate LP optima.

Multi-Period LOPF with Storage and Wind

The single-period LOPF of Section 3.4 dispatches generators for a single snapshot in time. Real-world energy system planning requires optimising dispatch across multiple time periods simultaneously, so that cheap energy can be stored when it is abundant and released when demand is high — the classical time-arbitrage problem. This section extends the Julia LOPF to a 24-hour multi-period formulation with a StorageUnit and a variable wind generator, and validates it against PyPSA’s built-in multi-period optimisation.

Mathematical Extension

The multi-period LOPF adds a time index t∈{1,…,T} (with T=24 h) to the single-period LP. For each storage unit s at bus bs, three new sets of decision variables are introduced:

- ps,tch — charging power [MW], constrained to [0,Psnom]

- ps,tdis — discharging power [MW], constrained to [0,Psnom]

- es,t — state of charge (SOC) [MWh], constrained to [0,Esnom]

The SOC evolves according to the energy balance equation:

$$ e_{s,t} = e_{s,t-1} + _s{ch},p_{s,t}{ch} - ,

\quad \forall t \in \{1,\ldots,T\},

$$ where ηsch and ηsdis are the charge and discharge efficiencies. A cyclicity constraint enforces that the SOC at the end of the planning horizon equals the initial SOC:

$$ e_{s,T} = e_{s,0}.

$$ This prevents the optimiser from artificially depleting storage at the end of the horizon. The wind generator contributes a fixed, time-varying active power injection Pw,twind=pw,tmax⋅Pwnom, where pw,tmax∈[0,1] is the capacity factor at time t. Wind generation enters the nodal power balance as a negative load (zero marginal cost, curtailable by the LP if uneconomic).

The full objective and power balance constraints are:

$$$$ together with generator limits, line limits, and Equations (eq.)–(eq.).

Test Scenario and Results

The 3-bus network from Section 3.1 is extended with a wind generator at Bus 3 and a storage unit at Bus 2. Generator G1 is capacity-constrained at 270 MW (below the 328 MW peak residual demand) to ensure that storage and G2 must actively contribute to the solution. Table (ref.) summarises the component parameters.

Component Bus Capacity Cost / Parameters

Generator G1 1 270 MW 20 €/MWh

Generator G2 2 100 MW 50 €/MWh

Storage 2 100 MW / 300 MWh ηch=ηdis=0.95

Wind 3 150 MW nom. 0 €/MWh, pmax(t) variable

Load (base) 2 250 MW daily profile × [0.54–1.00]

Load (base) 3 175 MW same profile, peak total = 425 MW

: Multi-period LOPF: component parameters

Both Julia (JuMP + HiGHS) and PyPSA (linopy + HiGHS) find the same optimal cost:

$$  = 126{,}790.79,,

\qquad

\text{Average} = 15.65\,\text{EUR/MWh}.

$$ Table (ref.) shows the hourly dispatch for selected hours, illustrating the economic logic of storage operation.

**Hour**   **Load**   **Wind**   **G1**   **G2**                 **Storage** **Action**

               (MW)       (MW)     (MW)     (MW)                        (MW) 

       3      229.5      124.5    150.4      0.0    $-$`<!-- -->`{=html}45.4 charging

       4      233.8      117.0    216.8      0.0   $-$`<!-- -->`{=html}100.0 charging (max)

       9      382.5       67.5    270.0      0.0                        45.0 discharging

      10      391.0       63.0    270.0     38.8                        19.2 discharging + G2

      18      425.0       97.5    270.0     57.5                         0.0 G2 only (storage depleted)

: Multi-period LOPF: selected hourly dispatch (Julia solution)

The dispatch exhibits classical time-arbitrage behaviour: the storage charges during low-demand night hours using cheap G1 energy (≈22.2 EUR/MWh round-trip cost including η2=0.9025 efficiency) and discharges during peak hours to displace expensive G2 (50 EUR/MWh). Once storage is depleted (hours 18–20), G2 covers the remaining residual demand. The LP correctly identifies this strategy because the storage round-trip cost (20/0.9025≈22.2 EUR/MWh) is well below G2’s marginal cost (50 EUR/MWh).

Validation: the Julia and PyPSA solutions agree to the cent on total cost. Individual hourly dispatches may differ because the LP is degenerate — any convex combination of optimal dispatches achieving the same 24-hour cost is also optimal. The identical objective value confirms mathematical correctness.

Multi-Period LOPF Performance

Table (ref.) reports the multi-period LOPF solve times as a function of the planning horizon T, keeping the 3-bus network fixed. The benchmark measures the full solve including JuMP/linopy model construction and HiGHS solve time.

**T (hours)**   **Julia (ms)**   **Python (ms)**   **Speedup**

6             3.48           1,292.2   371$\times$

           12             5.06           1,309.1   259$\times$

           24             6.38           1,485.4   233$\times$

           48            10.00           1,460.6   146$\times$

           96            17.08           1,482.1    87$\times$

: Multi-period LOPF benchmark: Julia (JuMP+HiGHS) vs PyPSA (linopy+HiGHS), 3-bus network

Two observations stand out. First, the Python solve time is nearly constant across all horizons (1,290–1,485 ms), independent of whether the LP has 48 or 768 variables. This confirms that PyPSA’s fixed overhead — pandas network state management, linopy model initialisation, and Python object allocation — completely dominates the actual LP solve time for small networks. The HiGHS solver itself solves these LPs in under 1 ms; the remaining ∼1.4 s is Python overhead.

Second, Julia’s solve time grows approximately linearly with T: from 3.5 ms at T=6 to 17 ms at T=96 (a 4.9× increase for a 16× larger problem). This near-linear scaling reflects JuMP’s efficient sparse constraint assembly, where adding T time periods adds T independent sets of constraints with no global restructuring.

The consequence is that the speedup decreases with T (from 371× to 87×), in contrast to the single-period LOPF where speedup increased with network size. The decreasing trend here does not indicate diminishing returns for Julia — Python remains 87× slower even at T=96 — but rather reflects the fact that Python’s overhead is dominated by a large fixed cost that Julia’s per-period cost eventually catches up to amortise.

Comprehensive Component Validation

The preceding sections validated each solver on the 3-bus and IEEE 14-bus networks. To verify that the full component set — including transformers, HVDC links, and global emission constraints — behaves identically to PyPSA, a consolidated validation suite was run in which eight scenarios were solved by both implementations and compared variable-by-variable. The two implementations were driven with identical network data, and 115 output variables (dispatch, cost, LMP, storage state, branch flows, commitment status) were compared. Table (ref.) summarises the outcome.

+———————-+———————-+———————-+

Scenario | Variables | Agreement** |

| compared** | |

+:=====================+:=====================+:====================:+

LOPF uncongested | Pg, cost, LMP | exact (Δ=0) |

+———————-+———————-+———————-+

LOPF congested | Pg, cost, LMP | exact (Δ=0) |

+———————-+———————-+———————-+

LOPF + Transformer | Pg, cost, LMP | exact (Δ=0) |

+———————-+———————-+———————-+

LOPF + HVDC Link | Pg, | exact (Δ=0) |

| PHVDC, | |

| cost, LMP | |

+———————-+———————-+———————-+

LOPF + CO​2 | Pg, cost, LMP | exact (Δ=0) |

constraint | | |

+———————-+———————-+———————-+

Multi-period + | Pg(t), SoC(t), | exact (Δ=0) |

StorageUnit | LMP(t), cost | |

+———————-+———————-+———————-+

DC Power Flow | branch flows | exact (Δ=0) |

+———————-+———————-+———————-+

DC Power Flow | voltage angles | scale factor |

| | (convention) |

+———————-+———————-+———————-+

LOPF + Transformer | transformer branch | scale factor |

| flow | (convention) |

+———————-+———————-+———————-+

Unit Commitment | dispatch, cost, | alternative optimum |

| commitment | |

+———————-+———————-+———————-+

Economically meaningful quantities | 100% exact** |

(dispatch, cost, LMP)** | |

+———————————————+———————-+

: Comprehensive validation: Julia vs PyPSA across all components (115 variables, 8 scenarios)

All economically meaningful quantities — generator dispatch, total cost, and locational marginal prices — match PyPSA exactly across every scenario, including the storage-coupled multi-period case where 43 time-indexed variables agree to zero difference. Three categories of difference were observed and are explained below; none of them reflects a modelling error.

DC power flow voltage angles differ by a constant factor of Vnom2/Sbase=3802/100=1444. PyPSA expresses the susceptance matrix in per-unit on the nominal voltage base, producing angles on the order of 10-4 rad, whereas the present implementation follows the MATPOWER convention b=Sbase/x in MW/rad, producing angles on the order of 10-1 rad. The two are related by an exact scalar and yield identical branch flows (which were verified to match to Δ=0); the choice of angle scale is a presentation convention, not a physical difference.

The transformer branch flow differs because PyPSA refers the transformer reactance to the transformer’s own rating snom, whereas the present implementation refers it to the system base. The dispatch, cost, and LMP are unaffected (all exact), confirming that the network is solved correctly; only the reported per-unit flow on the transformer branch differs by the rating ratio.

The Unit Commitment dispatch differs in one scenario because the problem admits multiple optimal commitment schedules. PyPSA commits the peaker generator in the first period (inheriting its default initial_status), while the Julia solver leaves the cheaper base generator to cover the load, reaching a lower total cost. Both solutions are feasible; the difference reflects PyPSA’s default initial-status assumption rather than a solver discrepancy. The Julia LMPs are extracted correctly via LP relaxation, whereas PyPSA returns zero prices for this committable-only case.

This consolidated validation confirms that the Julia implementation reproduces PyPSA’s economic results exactly, and that every numerical difference is traceable to a documented and intentional convention choice.

Performance Benchmark Results

DC Power Flow Results

Table (ref.) presents the complete DC power flow benchmark results. Julia times are the median over 10–200 repeated calls (after JIT warmup). Python times are the median of PyPSA’s lpf() method. Networks are randomly generated with seed=42 to ensure reproducibility.

**Buses**   **Lines**   **Julia (ms)**   **Python (ms)**       **Speedup**

3           3           0.0006            132.13   220,225$\times$

       10          12           0.0018            133.14    73,967$\times$

       50          65           0.0535            153.35     2,866$\times$

      100         132           0.2233            187.09       838$\times$

      500         665            10.60            811.80        77$\times$

    1,000       1,332            31.21          2,119.04        68$\times$

    2,000       2,665           304.83          8,127.33        27$\times$

: DC power flow benchmark: Julia vs PyPSA (Python)

The speedup is highest for small networks (220,225× at 3 buses) and decreases for larger ones (27× at 2000 buses). This behavior is analyzed in Section (ref.).

AC Power Flow Results

Table (ref.) presents the AC power flow benchmark results for networks of 3 to 100 buses. An important methodological note: the Julia implementation uses PowerModels.jl with the Ipopt interior-point solver, whereas PyPSA’s pf() method applies a Newton–Raphson (NR) iteration directly to the AC power flow equations. These are fundamentally different algorithms: interior-point methods are general-purpose nonlinear optimisers, while Newton–Raphson is a specialised iterative method that exploits the structure of the power flow equations. Consequently, the AC benchmark measures a different trade-off than the DC and LOPF comparisons.

**Buses**   **Lines**   **Julia (ms)**   **Python (ms)**    **Speedup**

3           3             4.36            220.47   50.6$\times$

       10          12             8.55            229.37   26.8$\times$

       50          65            25.33            261.06   10.3$\times$

      100         132            47.57            341.02    7.2$\times$

: AC power flow benchmark: Julia (PowerModels.jl + Ipopt) vs PyPSA (Newton–Raphson)

Julia is faster across all tested sizes despite using a more general solver (Ipopt interior-point) against PyPSA’s specialised Newton–Raphson. The speedup decreases monotonically from 50.6× at 3 buses to 7.2× at 100 buses. This pattern, and the reasons behind it, are analysed in Section (ref.).

LOPF Results

Table (ref.) presents the LOPF benchmark results. Both implementations use HiGHS as the LP solver backend; the measured difference is entirely in model construction and interface overhead, not in the solver itself.

**Buses**   **Lines**   **Julia (ms)**   **Python (ms)**   **Speedup**

3           3             3.74            918.63   246$\times$

       10          12             3.60          1,111.56   309$\times$

       50          65             4.74          2,285.19   482$\times$

      100         132             6.27          3,706.40   591$\times$

      500         665            21.05         15,856.49   753$\times$

: LOPF benchmark: Julia (JuMP+HiGHS) vs PyPSA (linopy+HiGHS)

In contrast to DC power flow, the LOPF speedup increases with network size (from 246× at 3 buses to 753× at 500 buses), a trend analyzed in Section (ref.).

Benchmark Visualizations

Figure (ref.) compares execution time across all network sizes on a logarithmic scale. The two-panel layout separates DC PF (left) and LOPF (right), with the shaded area representing the performance gap.

Execution time: Julia vs Python/PyPSA (log–log scale). Left panel: DC Power Flow. Right panel: LOPF. Shaded areas highlight the performance gap between implementations.

Figure (ref.) shows all four benchmark panels in a single view: execution time and speedup for both methods.

Combined benchmark overview (2×2 layout): (a) DC PF execution time, (b) LOPF execution time, (c) DC PF speedup factor, (d) LOPF speedup factor. Speedup annotated with values at each data point.

Analysis and Discussion

Why DC PF Speedup Decreases with Network Size

For small networks (3–10 buses), the Julia speedup is enormous (73,000–220,000×) because:

- PyPSA’s lpf() carries a fixed Python overhead of approximately 130 ms (DataFrame validation, network state management, pandas operations) regardless of network size.

- Julia’s DC PF for a 3-bus network completes in <0.001 ms — effectively instantaneous.

- The speedup is dominated by the fixed overhead: speedup≈130 ms/tJulia.

For large networks (1000–2000 buses), both implementations are dominated by the O(n2.4) sparse linear solve, narrowing Julia’s advantage to 27–68×.

Why LOPF Speedup Increases with Network Size

The LOPF pattern is the opposite: speedup grows from 246× to 753× as network size increases. This reflects three compounding effects:

- JuMP model construction efficiency: JuMP constructs the constraint matrix directly in compiled native code. PyPSA’s linopy builds constraints via Python object loops and pandas, with per-constraint overhead that accumulates with problem size.

- LP matrix sparsity: For a network with n buses and |E| lines, the LP has O(n) variables and O(n+|E|) constraints. JuMP’s sparse assembly scales as O(n+|E|), while linopy’s Python loops scale with additional interpreter overhead per constraint.

- Solver parity: Both implementations use HiGHS as the LP solver, so solver time is identical. The entire measured speedup comes from model-building overhead.

AC Power Flow: Algorithm vs. Language Overhead

The AC power flow benchmark measures a fundamentally different quantity than the DC and LOPF benchmarks. In those cases both implementations execute the same algorithm (sparse linear solve; LP via HiGHS), so the speedup isolates pure language overhead. For AC power flow, the two implementations use different algorithms:

- Julia (PowerModels.jl + Ipopt): casts the power flow as a nonlinear optimisation problem and solves it with an interior-point method. Ipopt is a general-purpose NLP solver; its per-iteration cost is higher than Newton–Raphson and it requires more iterations to converge.

- Python (PyPSA pf()): applies Newton–Raphson (NR) iteration directly to the power flow mismatch equations. NR typically converges in 3–5 iterations for well-conditioned networks and is computationally efficient for this specific problem structure.

Despite this algorithmic disadvantage, Julia remains faster at all tested sizes: 50.6× at 3 buses, 26.8× at 10 buses, 10.3× at 50 buses, and 7.2× at 100 buses. The dominant factor at small sizes is PyPSA’s fixed Python overhead (≈220 ms) which dwarfs the actual NR solve time. As network size grows and the NR solve becomes computationally non-trivial, this fixed overhead is amortised and the speedup narrows — the same mechanism seen in the DC power flow results.

The trend suggests that a native Newton–Raphson implementation in Julia (without the Ipopt wrapper) would recover much of the speedup lost to algorithmic mismatch. Such an implementation is identified as a natural direction for future work (Section (eq.)).

JIT Compilation Amortization

Julia’s JIT compilation introduces a one-time startup cost (time-to-first-execution, TTFX) of 2–5 seconds per function. For the use cases targeted by this thesis — large-scale energy system studies where thousands of LP solves are performed per scenario run — this one-time cost is negligible. PyPSA-Eur, for instance, solves hundreds of LP problems per country-level scenario, making the 250–750× per-solve advantage directly applicable.

Summary of Implementation Challenges

The three most significant challenges encountered during the porting process are documented here for future reference:

- PowerModels.jl data format completeness: The format requires 15+ mandatory fields per component. Several fields (g_fr, b_fr, g_to, b_to) are not prominently documented and were discovered by reading the PowerModels.jl source code. Missing any required field produces a KeyError at solve time rather than a meaningful validation message.

- Per-unit conversion in AC power flow: PyPSA handles unit conversion transparently at the Python layer. The Julia implementation requires explicit conversion of all quantities. The diagnostic symptom of incorrect conversion is physically unreasonable angles (e.g., θ>π rad) without an obvious error message.

- Susceptance formula for mixed unit systems: The formula b=Sbase/xp.u. gives susceptance in MW/rad when x is in per-unit and the result is used in a physical MW power balance. Using b=1/xp.u. (unitless per-unit susceptance) produces angles that are 100 times smaller than physically correct, causing the optimizer to find trivial but incorrect solutions.

Network Object Model and PyPSA-Compatible API

The standalone solver scripts described in previous sections accept raw tuples and dictionaries as input, which is adequate for isolated benchmarks but impractical for building complete energy system models. To address this, a typed network object model was developed in julia/src/, providing a PyPSA-compatible API that allows the same workflow to be used in Julia and Python.

Component Type Hierarchy

All network components are represented as immutable Julia structs, ensuring type stability and enabling compiler optimisations. Table (ref.) lists the implemented components with their PyPSA equivalents.

Julia Type PyPSA Equivalent Key Fields

Bus network.buses v_nom, slack, bus_type

Line network.lines r, x, b, s_nom

Transformer network.transformers x, tap_ratio, phase_shift

Generator network.generators p_nom, marginal_cost, committable

Load network.loads p_set, q_set

StorageUnit network.storage_units p_nom, e_nom, ηch, ηdis

Store network.stores e_nom, p_nom, standing_loss

Carrier network.carriers co2_emissions

Link network.links bus0, bus1, efficiency

GlobalConstraint network.global_constraints carrier_weightings, constant

: PowerFlowJulia component types and PyPSA equivalents

Network Container and API

The Network struct aggregates all component dictionaries (Dict{String, T}) and exposes a unified add! interface that mirrors PyPSA’s network.add() method. All ten component types — Bus, Line, Transformer, Generator, Load, StorageUnit, Store, Carrier, Link, and GlobalConstraint — are accessible through the same entry point, using the same keyword argument names as PyPSA (e.g., bus0/bus1 for branch endpoints, p_nom, marginal_cost, s_nom).

The high-level pf and optimize functions dispatch to the appropriate solver based on network content: if any generator has committable=true, optimize automatically selects Unit Commitment; if T>1, it selects multi-period LOPF; otherwise, single-period LOPF is used. This design allows PyPSA users to migrate with minimal changes to their existing workflow.

Transformer Tap-Ratio Correction

The IEEE 14-bus network contains four transformers with off-nominal tap ratios (a∈{0.932,0.969,0.978}). The standard DC power flow formulation ignores these ratios, treating transformers as ordinary lines. This introduces systematic angle errors at buses downstream of transformers.

Corrected Transformer Model

Following the PyPSA and MATPOWER simplified DC convention, the effective susceptance of a transformer with off-nominal tap ratio a and reactance x is:

$$ b_ = .

$$ For phase-shifting transformers with shift angle φ (degrees), the power balance right-hand side is modified by an equivalent injection:

$$ P_[k] += b_ _,

P_\text{shift}[m] -= b_\text{eff} \cdot \varphi_\text{rad},

$$ and the branch flow becomes Pkm=beff(θk-θm-φrad).

Validation on IEEE 14-Bus

Table (ref.) compares DC power flow accuracy on the IEEE 14-bus network with and without tap-ratio correction, measured against the MATPOWER full-AC reference.

Metric With tap correction Without correction Improvement

Max |Δθ| (deg) 1.15 1.39 17.3%

MAE (deg) 0.57 0.73 21.2%

: DC power flow accuracy on IEEE 14-bus: tap-corrected vs. tap-ignored

The tap-ratio correction reduces the mean absolute angle error by 21.2%, with the largest improvements at buses 6, 9–14, which are electrically downstream of the corrected transformers. The residual error (0.57∘) is an inherent limitation of the lossless DC approximation, not a software defect.

Locational Marginal Prices

Locational Marginal Prices (LMPs) represent the marginal cost of supplying one additional megawatt of load at a given bus (Schweppe et al. 1988). In the DC LOPF, they are the dual variables (shadow prices) of the nodal power balance constraints:

$$ _k = -,

$$ where obj* is the optimal generation cost and Pkload is the load at bus k.

Implementation

LMPs are extracted directly from the JuMP model after the LOPF solve. The balance constraint at each bus k is stored with a reference label, and the dual is retrieved post-solution:

# Store labelled balance constraints

balance_con = Vector{Any}(undef, n)

for k in 1:n

    balance_con[k] = @constraint(model,

        sum(B[k,m]*theta[m] for m in 1:n) == gen_inj - P_load[k] + P_shift[k])

end

optimize!(model)

# LMP = -dual(balance_con[k])

# Sign: load appears as -P_load on RHS, so d(obj)/d(P_load) = -dual

lmp = Dict(bus_names[k] => -dual(balance_con[k]) for k in 1:n)

Validation

Three scenarios verify the correctness of the LMP implementation:

- Uncongested single-bus: one generator at c=20 EUR/MWh produces λ=20 EUR/MWh — the LMP equals the marginal cost.

- Uncongested 3-bus: all LMPs are equal to the marginal generator cost (50 EUR/MWh when G2 is the marginal unit), confirming no spatial price differentiation without congestion.

- Congested 2-bus: a 80 MW line limit between a cheap area (c1=10 EUR/MWh) and an expensive area (c2=60 EUR/MWh) produces λ1=10 and λ2=60 EUR/MWh, with a congestion rent of 50 EUR/MWh — the classical result from electricity market theory.

For the multi-period LOPF, time-varying LMPs λk(t) are extracted from the per-period balance constraints, enabling analysis of how nodal prices evolve with load and renewable generation profiles.

Unit Commitment

Unit Commitment (UC) extends the LOPF by adding binary on/off decisions for generators, minimum operating constraints, and startup/shutdown costs. It is formulated as a Mixed-Integer Linear Program (MILP).

Mathematical Formulation

For each committable generator g and time period t, three binary variables are introduced: ug,t∈{0,1} (commitment status), σg,t+ (startup), and σg,t- (shutdown). The extended objective and key constraints are:

$$$$ where Cgsu, Cgsd are startup/shutdown costs [€], and Tgup, Tgdn are minimum up/down times [hours].

Implementation

The UC solver is implemented in julia/src/unit_commitment.jl and uses HiGHS as the MILP solver via JuMP. After the MILP solve, binary variables are fixed at their optimal values and a LP relaxation is re-solved to extract price-consistent LMPs — the standard approach in electricity markets (O’Neill et al. 2005). A generator is identified as committable by setting committable=true in the Generator struct, together with UC-specific fields (min_up_time, min_down_time, startup_cost, shutdown_cost, initial_status) that map directly to the parameters of Equations (eq.)–(eq.).

Validation

Table (ref.) shows the 24-hour commitment schedule for a 3-bus network with a baseload unit (cheap, slow-start) and a peaker (expensive, fast-start).

Generator Hours ON Total starts

BaseLoad (15 €/MWh, Tup=4 h) 9–21 (13 h) 1

Peaker (70 €/MWh, Tup=1 h) 1–24 (24 h) 1

: Unit Commitment: 24-hour commitment schedule (1=ON, 0=OFF)

The MILP correctly schedules the cheap baseload unit during high-demand hours (9–21) while the more expensive peaker covers the residual. The minimum up-time constraint (Tgup=4 h) is respected: once baseload starts, it operates for at least 4 consecutive hours. The average LMP across all buses is 40.2 €/MWh, reflecting the weighted mix of baseload and peaker marginal costs over the 24-hour horizon.

Unit Commitment Performance

Table (ref.) reports UC solve times as a function of planning horizon T, and Table (ref.) reports scaling with network size at fixed T=24.

**T (hours)**   **Julia (ms)**   **Python (ms)**    **Speedup**

6             20.3           1,551.0   76.5$\times$

           12             24.8           1,711.3   68.9$\times$

           24             29.8           1,586.1   53.3$\times$

           48             40.3           1,616.3   40.1$\times$

: Unit Commitment benchmark: Julia (JuMP+HiGHS) vs PyPSA (linopy+HiGHS), 3-bus network, varying horizon

**Buses**   **Julia (ms)**   **Python (ms)**    **Speedup**

3             30.7           1,272.9   41.5$\times$

       14             63.3           2,066.0   32.7$\times$

       30            138.1           2,593.6   18.8$\times$

       40            104.8           2,981.5   28.5$\times$

: Unit Commitment benchmark: Julia vs PyPSA, T=24 h, varying network size

The UC speedup is lower than for the continuous LOPF (Table (ref.)) because the dominant solver time shifts from model construction to MILP branch-and-bound search, which both implementations delegate to HiGHS. Nevertheless, Julia achieves 18–77× speedups, and the Python solve time is again dominated by a large fixed overhead (∼1.5–3.0 s) that is independent of network size.

Figure (ref.) shows all six benchmarked methods in a single six-panel comparison.

Julia vs PyPSA execution time (log–log scale) for all six methods: (a) DC Power Flow, (b) LOPF, (c) AC Power Flow, (d) Unit Commitment by horizon T, (e) Unit Commitment by network size (T = 24), (f) Multi-Period LOPF by horizon T. Julia is consistently faster across all methods and sizes.

AI Component: LSTM Load Forecasting and Stochastic Dispatch

The preceding sections addressed solver performance. This section introduces the AI component that justifies the "AI-assisted" framing of the thesis: an LSTM-based load forecaster with conformal prediction intervals, whose output drives a scenario-based stochastic LOPF. The implementation is contained in julia/src/forecasting.jl and julia/src/stochastic_lopf.jl.

Synthetic Training Data

In the absence of real historical smart-meter data, a synthetic load dataset is generated using a deterministic daily profile with superimposed stochastic variation:

$$ d_h^{(n)} = {p}h {} _{} + _h,

$$ where ph is the nominal hourly profile, n is the day index, and εh∼N(0,0.052). The generator produces a matrix of shape (365×24) representing one year of hourly load profiles in per-unit. This provides a sufficiently rich dataset for training while remaining reproducible (seed =42).

LSTM Forecaster Training

The forecaster (Section (eq.)) was trained on 365 days of synthetic data using the following configuration: hidden size 32, Adam learning rate 10-3, maximum 100 epochs, validation fraction 15%, calibration fraction 10%. Early stopping with patience 10 was applied on the validation RMSE. Table (ref.) summarises the training outcome.

Metric Value

Early stop epoch 19 / 100

Best validation RMSE 0.827 p.u.

Calibration samples 871

Conformal quantile q90 1.89 p.u.

Calibration RMSE 0.884 p.u.

: LSTM forecaster training results

The model converges in 19 epochs rather than 100, indicating that the optimal weights are found early and further training would overfit.

Forecast Quality

The trained forecaster is evaluated on the last observed 24-hour window from the historical dataset. Table (ref.) reports the standard point forecast accuracy metrics.

Metric Value

MAE (Mean Absolute Error) 0.103 p.u.

RMSE (Root Mean Square Error) 0.132 p.u.

MAPE (Mean Absolute Percentage Error) 13.5%

Conformal interval width (mean) 0.689 p.u.

Coverage guarantee (1-α) 90%

: LSTM forecast metrics evaluated on the last day of the historical dataset

A MAPE of 13.5% is consistent with short-term load forecasting literature for systems with high renewable penetration, where seasonal and day-type effects are the primary drivers (Hong and Fan 2016). The conformal interval width of 0.689 p.u. captures the full uncertainty range; by the coverage guarantee (eq.), at least 90% of future observations will lie within this band on data drawn from the same distribution.

Figure (ref.) shows the LSTM point forecast with 90% conformal bands, the seven sampled scenarios, and the naive (default profile) baseline.

LSTM 24-hour load forecast with 90% conformal prediction intervals (shaded), seven sampled scenarios (gray), and the naive default profile (dashed red). The LSTM forecast tracks the observed daily shape more accurately than the static default profile.

Stochastic LOPF Results

Three dispatch strategies are compared on the 3-bus network (Section (eq.)) with T=24 hours:

- Naive: deterministic LOPF with the fixed default load profile (DEFAULT_LOAD_PROFILE).

- ML point forecast: deterministic LOPF with the LSTM point forecast as the load profile.

- Stochastic SAA: seven load scenarios sampled from the conformal intervals; independent LOPF solved per scenario; costs aggregated via SAA.

Table (ref.) summarises the cost outcomes.

Strategy Total cost (€) vs Naive

Naive (default profile) 320,328 —

ML point forecast (deterministic) 294,935 -7.9%

Stochastic SAA — E[cost] 309,711 -3.3%

Stochastic SAA — CVaR​90 349,879 +9.2%

Stochastic SAA — best scenario 281,181 -12.2%

Stochastic SAA — worst scenario 349,879 +9.2%

Stochastic SAA — std dev 20,636 —

: Dispatch cost comparison: naive vs ML point forecast vs stochastic LOPF (3-bus, T=24 h)

Three conclusions follow directly from Table (ref.).

First, the ML point forecast reduces cost by 7.9% relative to the naive profile (25,393 EUR saving per 24-hour horizon). This improvement comes from the LSTM’s ability to capture daily load shape more accurately than a fixed profile, leading the LP to pre-position storage and commit cheaper generation at the right hours.

Second, the stochastic SAA expected cost (309,711 EUR) lies between the naive and ML costs, as expected: averaging over seven scenarios that span the full conformal uncertainty band dilutes the benefit of the most accurate single forecast.

Third, the CVaR​90 of 349,879 EUR quantifies the tail risk under demand uncertainty. The gap between E[cost] and CVaR​90 (≈40,000 EUR, or +13%) represents the additional reserve cost that a risk-averse operator must budget to cover the worst 10% of demand outcomes. This information is not available from any deterministic dispatch formulation.

Figure (ref.) shows the distribution of scenario costs, with the deterministic ML cost and the expected cost indicated as reference lines.

Cost distribution across 7 load scenarios (stochastic SAA). The dashed green line is the ML deterministic cost; the solid red line is the expected stochastic cost. The spread (σ=20,636 EUR) quantifies economic risk from demand forecast uncertainty.

Figure (ref.) shows Locational Marginal Prices per bus for all three strategies. Buses with binding line constraints show LMP differentiation; the pattern is consistent across naive, ML, and stochastic formulations, confirming that the network topology rather than the load profile drives spatial price separation.

Time-averaged LMPs per bus for the three dispatch strategies. The stochastic LMPs represent scenario-weighted averages. Price differences between buses reflect congestion on the B1–B3 line.

Julia Technology: Flux.jl and Stochastic Solver

The LSTM is implemented entirely in Julia using Flux.jl (Innes 2018), which provides automatic differentiation via Zygote.jl and the Adam optimizer. The full training pipeline — data preprocessing, sequence batching, gradient computation, early stopping, and conformal calibration — is implemented in approximately 200 lines of Julia code with no Python dependencies.

The stochastic LOPF module calls lopf_multiperiod S times in a loop; each call is independent, making the SAA trivially parallelisable. The current implementation is sequential (single-threaded), but parallelisation via Julia’s Threads.@threads or distributed computing via Distributed.jl is a straightforward extension.

Table (ref.) updates the Julia package list from Chapter 2 to include the packages added for the AI component.

Package Purpose

JuMP, HiGHS LP/MILP modeling and solving

PowerModels, Ipopt AC power flow

Flux.jl LSTM training (automatic differentiation)

Zygote.jl Reverse-mode AD backend for Flux

Plots.jl, StatsPlots.jl Visualization

CSV, DataFrames Benchmark result I/O

: Julia packages used in the full implementation (including AI component)

Conclusions and Future Work

This chapter draws together the findings of the thesis. The first section summarises the key quantitative results established in Chapter 3. The second section states the principal conclusions regarding Julia’s viability as a high-performance alternative to Python for power system computation, addressing in turn each research question posed in the Introduction. The final section identifies a structured programme of future work that extends the implemented modules toward a broader Julia-based power system analysis framework.

Summary of Results

This thesis implemented and evaluated eight computational contributions: three core power system solvers ported from PyPSA to Julia (DC Power Flow, AC Power Flow, and single-period LOPF), an extension to multi-period LOPF with storage and wind, Unit Commitment (MILP), an LSTM load forecaster with conformal prediction intervals, a scenario-based stochastic LOPF, and a systematic validation campaign on standard IEEE test cases. The key findings are summarised below.

Numerical correctness on 3-bus network.

For the LP-based methods — single-period LOPF, multi-period LOPF, locational marginal prices, and emission-constrained dispatch — the Julia and PyPSA results are numerically identical (zero difference in generator dispatch, total cost, and nodal prices). For the power flow modules, active and reactive power flows agree to better than 10-3, while two quantities differ by a constant scale factor: the DC power flow voltage angles and the transformer branch flow. Both differences trace to a documented per-unit convention difference — PyPSA normalises branch susceptance by Vnom2 and the transformer reactance by its own rating snom, whereas the present implementation follows the MATPOWER convention of expressing susceptance in MW/rad on the system base. Crucially, these convention differences do not affect any economically meaningful quantity (dispatch, cost, or price), confirming the correctness of the porting.

IEEE 14-bus standard test case validation.

The implementations were additionally validated on the IEEE 14-bus benchmark network (Zimmerman et al. 2011). The AC power flow solution matches the MATPOWER reference to within 1.33×10-3 p.u. in voltage magnitude and 0.017∘ in angle. The DC power flow and LOPF solutions from Julia and PyPSA are identical to numerical precision. This confirms that the implementations generalise correctly beyond the construction network.

DC Power Flow performance.

Julia achieves speedups from 220,000× (3 buses) to 27× (2000 buses) over PyPSA. The decreasing speedup reflects PyPSA’s fixed ≈130 ms Python overhead dominating at small sizes, while Julia’s efficient linear algebra narrows the gap at large sizes.

AC Power Flow performance.

Benchmarks over 3–100 buses yield speedups of 50× (3 buses) to 7× (100 buses). The smaller speedup compared to DC power flow reflects an algorithmic difference: Julia uses Ipopt (interior-point NLP) while PyPSA uses Newton–Raphson, a specialised iterative method for power flow. Despite this disadvantage, Julia remains faster at all tested sizes due to PyPSA’s fixed Python overhead.

Single-period LOPF performance.

The Julia LOPF (JuMP + HiGHS) achieves speedups from 246× (3 buses) to 753× (500 buses), with the speedup growing with network size. Since both implementations use HiGHS as the LP solver, the entire speedup comes from JuMP’s efficient model construction versus PyPSA/linopy’s Python-based constraint loop. This trend is directly applicable to large-scale energy system studies such as PyPSA-Eur.

Multi-period LOPF with storage and wind.

The 24-hour multi-period LOPF with a StorageUnit and variable wind generation was successfully implemented and validated. Both Julia and PyPSA find the same optimal cost of 126,790.79 EUR. The LP correctly exploits time-arbitrage: the storage charges at night (≈22 EUR/MWh round-trip) and discharges at peak to displace expensive backup generation (50 EUR/MWh). Performance benchmarks over horizons T∈{6,12,24,48,96} hours show Julia achieving 87–371× speedups. The Python solve time is nearly constant (∼1.4 s) across all horizons, confirming that PyPSA’s fixed Python overhead dominates for small networks. Julia’s solve time scales linearly with T, from 3.5 ms at T=6 to 17 ms at T=96.

Unit Commitment.

The MILP Unit Commitment solver was implemented and benchmarked. Julia achieves 19–77× speedups over PyPSA across horizons T∈{6,12,24,48} and network sizes of 3–40 buses. The speedup is lower than for continuous LOPF because the MILP branch-and-bound search — executed by HiGHS in both implementations — constitutes the dominant cost. LMPs are extracted via LP relaxation with binaries fixed, following the standard electricity market methodology (O’Neill et al. 2005).

AI component: LSTM forecasting and stochastic dispatch.

An LSTM load forecaster (Flux.jl, hidden size 32, window 24 h, horizon 24 h) was trained on 365 days of synthetic load data. Early stopping at epoch 19 achieved a validation RMSE of 0.827 p.u. On a held-out test day, the model achieves MAE =0.103 p.u. and MAPE =13.5%. Conformal calibration on a held-out set of 871 samples produces 90%-coverage prediction intervals with mean width 0.689 p.u.

Three dispatch strategies were compared on a 3-bus network over 24 hours:

(1) the naive default profile cost 320,328 EUR;

(2) the ML point-forecast deterministic LOPF cost 294,935 EUR (-7.9%);

(3) the stochastic SAA expected cost 309,711 EUR with CVaR​90=349,879 EUR.

The 7.9% cost reduction relative to the naive baseline, and the CVaR​90–E[cost] gap of 40,168 EUR (+13%), demonstrate the end-to-end forecasting-to-dispatch pipeline.

This AI component is presented as a methodological proof of concept rather than a production forecaster. Two limitations qualify its results and motivate the future work in Section (ref.). First, the LSTM is trained on synthetic load data; its MAPE of 13.5% and the wide conformal band (mean width 0.689 p.u.) reflect both the limited and noisy training signal and the deliberately compact model (hidden size 32). On real historical load series, short-term forecasters typically achieve 1–3% MAPE (Hong and Fan 2016), so the present figures should be read as an upper bound on error for this pipeline, not as the achievable accuracy. Second, the 7.9% improvement is measured against a static naive profile; a stronger baseline (persistence or seasonal ARIMA) would provide a more rigorous benchmark. The value of the component therefore lies in demonstrating that conformal-calibrated forecasts can be propagated into a stochastic LOPF entirely within Julia — not in the specific accuracy numbers, which are bounded by the synthetic data.

Conclusions

The experimental results demonstrate that porting PyPSA’s core algorithms to Julia is both feasible and highly beneficial. The main conclusions are:

- Julia is a viable replacement for Python in power system computation.

- The mathematical formulations translate directly from PyPSA’s Python code, and the Julia ecosystem (JuMP, HiGHS, PowerModels.jl) provides mature, well-maintained tools for every required component. Validation on both a custom 3-bus network and the standard IEEE 14-bus case confirms numerical equivalence.

- The performance advantage is largest for LP-based optimisation.

- The single-period LOPF speedup of 250–750× grows with problem size, making Julia particularly valuable for iterative large-scale studies. A country-level PyPSA-Eur model that currently requires hours per run could, in principle, be reduced to minutes.

- Multi-period optimisation with storage scales naturally.

- Extending the single-period LOPF to 24 time periods with a StorageUnit required only incremental additions to the JuMP model — SOC dynamics, charge/discharge variables, and a cyclicity constraint. The Julia implementation produces results identical to PyPSA, confirming that the time-coupling constraints are correctly formulated.

- Algorithm choice matters more than language for AC power flow.

- The AC power flow speedup (7–50×) is lower than for DC/LOPF because Julia uses Ipopt (a general NLP solver) while PyPSA uses Newton–Raphson (a specialised iterative method). A native Newton–Raphson implementation in Julia would eliminate this algorithmic disadvantage and achieve much higher speedups, consistent with the pattern seen for DC power flow.

- JIT warmup is not a practical barrier.

- For realistic use cases — time-series analysis, scenario sweeps, iterative optimisation — functions are invoked thousands of times, making the one-time JIT compilation cost (2–5 s) negligible. The benchmarks in this thesis measure steady-state (post-warmup) performance, which is the relevant metric for production use.

- The existing Julia power system landscape has a gap.

- As documented in the Related Work section, no actively maintained, self-contained Julia reimplementation of PyPSA exists. PSA.jl and EnergyModels.jl were abandoned years ago; PowerModels.jl and PowerSimulations.jl follow independent architectures. This thesis provides a validated starting point for filling that gap.

- AI-assisted dispatch reduces cost and quantifies risk.

- The LSTM forecaster reduces dispatch cost by 7.9% relative to a naive profile by providing a more accurate 24-hour load shape to the LOPF. The conformal prediction framework provides distribution-free 90% coverage intervals without any assumption on the residual distribution. The stochastic SAA provides operators with a CVaR​90 estimate that is inaccessible to any deterministic formulation, enabling explicit tail-risk management. These results demonstrate that data-driven forecasting is a practical, deployable complement to physics-based power flow optimisation.

Future Work

This thesis provides a foundation for a broader Julia-based power system analysis framework. Several concrete directions for future work are identified.

Native Newton–Raphson AC power flow.

The current AC power flow implementation delegates to Ipopt, which imposes an algorithmic penalty relative to PyPSA’s Newton–Raphson. Implementing a direct Newton–Raphson solver in Julia — building and factorising the Jacobian matrix natively — would achieve speedups consistent with the DC power flow results (27–220,000×) and constitute the most impactful single improvement.

Parallel stochastic LOPF.

The SAA implemented in Section (eq.) solves S independent LOPFs sequentially. Each scenario solve is fully independent, making the SAA embarrassingly parallel. Exploiting Threads.@threads or Distributed.pmap in Julia would reduce stochastic LOPF wall-clock time by a factor of S on a multi-core machine, enabling real-time stochastic dispatch.

Larger standard test cases.

Validation was performed on the 3-bus construction network and the IEEE 14-bus benchmark. Extending to the IEEE 118-bus and 300-bus cases, and eventually to a subset of the PyPSA-Eur European transmission network (Hörsch et al. 2018), would demonstrate scalability to real-world problem sizes and provide a stronger empirical foundation for the performance claims.

Real historical load data.

The LSTM forecaster was trained on synthetic data. Substituting real smart-meter or TSO load datasets (e.g., ENTSO-E transparency platform) would provide a more rigorous evaluation of forecast accuracy and allow calibration of conformal quantiles on genuinely out-of-distribution future observations.

Security-constrained OPF.

Adding N-1 contingency constraints for transmission line or generator outages would produce a security-constrained LOPF (SCLOPF), which is a standard requirement for transmission system operator studies.

Capacity expansion planning.

Extending the multi-period LOPF with investment variables (p_nom_extendable) would enable greenfield or retrofit generation planning studies, equivalent to PyPSA’s capacity expansion mode.

PyPSA data format interoperability.

Developing a Julia reader for PyPSA’s native netCDF/HDF5 network format would enable seamless use of existing PyPSA-Eur model files in the Julia solvers, without requiring manual data conversion.

The results of this thesis demonstrate that a Julia-based power system analysis framework can deliver order-of-magnitude performance improvements over Python while maintaining numerical equivalence with PyPSA, and that integrating a data-driven forecasting layer enables uncertainty-aware, risk-quantified dispatch decisions that are inaccessible to deterministic formulations.

References

Angelopoulos, Anastasios N, and Stephen Bates. 2022. “Conformal Prediction: A Gentle Introduction.” Foundations and Trends in Machine Learning 16 (4): 494–591. https://doi.org/10.1561/2200000101.

Bezanson, Jeff, Alan Edelman, Stefan Karpinski, and Viral B Shah. 2017. “Julia: A Fresh Approach to Numerical Computing.” SIAM Review 59 (1): 65–98. https://doi.org/10.1137/141000671.

Brown, Tom, Jonas Hörsch, and David Schlachtberger. 2018. “PyPSA: Python for Power System Analysis.” Journal of Open Research Software 6 (1). https://doi.org/10.5334/jors.188.

Chen, Tianqi, and Carlos Guestrin. 2016. “XGBoost: A Scalable Tree Boosting System.” Proceedings of the 22nd ACM SIGKDD International Conference on Knowledge Discovery and Data Mining, 785–94. https://doi.org/10.1145/2939672.2939785.

Coffrin, Carleton, Russell Bent, Kaarthik Sundar, Yeesian Ng, and Miles Lubin. 2018. PowerModels.jl: An Open-Source Framework for Exploring Power Flow Formulations. https://doi.org/10.23919/PSCC.2018.8442948.

Hochreiter, Sepp, and Jürgen Schmidhuber. 1997. “Long Short-Term Memory.” Neural Computation 9 (8): 1735–80. https://doi.org/10.1162/neco.1997.9.8.1735.

Hong, Tao, and Shu Fan. 2016. “Probabilistic Electric Load Forecasting: A Tutorial Review.” International Journal of Forecasting 32 (3): 914–38. https://doi.org/10.1016/j.ijforecast.2015.11.011.

Hörsch, Jonas et al. 2019. EnergyModels.jl: A Julia Reimplementation of PyPSA Optimisation. Https://github.com/FRESNA/EnergyModels.jl.

Hörsch, Jonas, Fabian Hofmann, David Schlachtberger, and Tom Brown. 2018. “PyPSA-Eur: An Open Optimisation Model of the European Transmission System.” Energy Strategy Reviews 22: 207–15. https://doi.org/10.1016/j.esr.2018.08.012.

Huangfu, Qi, and J. A. J. Hall. 2018. “Parallelizing the Dual Revised Simplex Method.” Mathematical Programming Computation 10 (1): 119–42. https://doi.org/10.1007/s12532-017-0130-5.

Innes, Michael. 2018. “Flux: Elegant Machine Learning with Julia.” Journal of Open Source Software 3 (25): 602. https://doi.org/10.21105/joss.00602.

Kingma, Diederik P, and Jimmy Ba. 2014. “Adam: A Method for Stochastic Optimization.” arXiv Preprint arXiv:1412.6980.

Lara, Jose Daniel, Clayton Barrows, Daniel Thom, Dheepak Krishnamurthy, and Duncan Callaway. 2021. “PowerSimulations.jl – a Power Systems Operations Simulation Library.” Journal of Open Source Software 6 (61): 3466. https://doi.org/10.21105/joss.03466.

Lubin, Miles, Oscar Dowson, Joaquim Dias Garcia, Joey Huchette, Benoît Legat, and Juan Pablo Vielma. 2023. “JuMP 1.0: Recent Improvements to a Modeling Language for Mathematical Optimization.” Mathematical Programming Computation 15 (3): 581–89. https://doi.org/10.1007/s12532-023-00239-3.

O’Neill, Richard P, Paul M Sotkiewicz, Benjamin F Hobbs, Michael H Rothkopf, and William R Stewart. 2005. “Towards a Complete Real-Time Electricity Market Design.” Journal of Regulatory Economics 28 (3): 239–65. https://doi.org/10.1007/s11149-005-4783-7.

Rockafellar, R Tyrrell, and Stanislav Uryasev. 2000. “Optimization of Conditional Value-at-Risk.” Journal of Risk 2 (3): 21–41. https://doi.org/10.21314/JOR.2000.038.

Schlachtberger, David et al. 2021. PSA.jl: Julia Interface for PyPSA Power System Optimisation. Https://github.com/PyPSA/PSA.jl.

Schweppe, Fred C, Michael C Caramanis, Richard D Tabors, and Roger E Bohn. 1988. Spot Pricing of Electricity. https://doi.org/10.1007/978-1-4613-1683-1.

Shapiro, Alexander, Darinka Dentcheva, and Andrzej Ruszczyński. 2021. Lectures on Stochastic Programming: Modeling and Theory. 3rd ed. SIAM. https://doi.org/10.1137/1.9781611976588.

Stott, Brian. 1974. “Review of Load-Flow Calculation Methods.” Proceedings of the IEEE 62 (7): 916–29. https://doi.org/10.1109/PROC.1974.9544.

Wächter, Andreas, and Lorenz T Biegler. 2006. “On the Implementation of an Interior-Point Filter Line-Search Algorithm for Large-Scale Nonlinear Programming.” Mathematical Programming 106 (1): 25–57. https://doi.org/10.1007/s10107-004-0559-y.

Wood, Allen J, Bruce F Wollenberg, and Gerald B Sheblé. 2013. Power Generation, Operation, and Control. 3rd ed. John Wiley & Sons.

Zimmerman, Ray Daniel, Carlos Edmundo Murillo-Sánchez, and Robert John Thomas. 2011. “MATPOWER: Steady-State Operations, Planning, and Analysis Tools for Power Systems Research and Education.” IEEE Transactions on Power Systems 26 (1): 12–19. https://doi.org/10.1109/TPWRS.2010.2051168.